3GPP TSG-SA WG2 Meeting #158	S2-2309074

# Goteborg, Sweden, August 21 – 25, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4842 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of the Unavailability Period during Network congestion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SUECR |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the incoming LS from CT1 (S2-2308294), they are unclear about the UE behaviour - when the UE’s request to activate the Unavailability Period is rejected with cause value #22, congestion; and - when, upon upper layers request to activate Unavailability Period, a backoff timer (e.g. T3346) is running in the UE. The UE is allowed to trigger the MRU and degestration procedure to activate the Unavailability Period, however when the MM back off timer is running, the UE is only allowed to trigger the deregistration procedure. Therefore if the MM back off timer is running at the UE or it was rejected by the network due to congestion control, it is proposed that the UE can perform deregistration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the MM back off timer is running at the UE or it was rejected by the network due to congestion control, the UE can perform deregistration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour for activating unavailability period is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.4.1.4	Support of Unavailability Period

During Registration procedure, the UE supporting the Unavailability Period feature provides "Unavailability Period Support" indication as part of 5GMM Core Network Capability in Registration Request message for initial registration and for every mobility registration. The AMF indicates whether the corresponding feature is supported in the AMF by providing the "Unavailability Period Supported" indication in Registration Accept message.

If the UE and network support Unavailability Period and an event is triggered in the UE that would make the UE unavailable or lose coverage (see clause 5.4.13.1) for a certain period of time, the UE uses Support of Unavailability Period to inform the AMF of the upcoming unavailability and whether it is due to NR satellite access discontinuous coverage. Use of Support of Unavailability Period for loss of coverage due to NR satellite access discontinuous coverage shall only be used if both UE and the AMF signalled "discontinuous coverage support", see clause 5.4.13.0.

If the use of Support of Unavailability Period is not due to NR satellite access discontinuous coverage, the UE may store its MM and SM context in the USIM or Non-Volatile memory in the ME to be able to reuse it after its unavailability period. If the UE can store its contexts and the Mobility Management back-off timer is not running, the UE may trigger Mobility Registration Update procedure; otherwise the UE shall trigger UE-initiated Deregistration procedure.

NOTE:	How and where the UE stores its contexts depends upon the UE implementation and the type of unavailability. The UE can store some or all of its contexts in the ME or USIM using existing ME or USIM functionality.

Before the start of an event that makes the UE unavailable, the UE includes an indication and type of unavailability, the Start of the Unavailability Period if known and the Unavailability Period Duration (if known) and triggers either Mobility Registration Update or UE initiated Deregistration procedure:

a)	If the UE initiates Mobility Registration Update procedure:

1)	If the UE did not include a Start of Unavailability Period, the AMF considers implicitly the Start of Unavailability Period to be the time at which it has received the Registration Request message from the UE. If the UE included a Start of Unavailability Period, the Start of Unavailability Period indicates the time at which the UE determines it expects to be unavailable, i.e. time until which the UE determines it is available.

2)	The AMF may determine, if not provided by the UE, or update the Unavailability Period Duration.

If the AMF knows an Unavailability Period Duration (e.g. based on the type of unavailability and other information available to the AMF as described in clause 5.4.13.3), and the UE did not include an Unavailability Period Duration or the UE included an Unavailability Period Duration different to the Unavailability Period Duration known to the AMF, the AMF may use either the Unavailability Period Duration known to the AMF or the Unavailability Period Duration from the UE as the Unavailability Period Duration. The AMF should include the Unavailability Period Duration known to the AMF in the Registration Accept. How the UE treats the AMF provided Unavailability Period Duration is up to UE implementation e.g. to help to determine when to return to coverage after a discontinuous coverage period, whether to listen to paging in eDRX, not to initiate any NAS signalling (including Service Request for MO data) within the discontinuous coverage period in case of any UL signalling/data request or the UE may deactivate its Access Stratum functions for NR satellite access in order to optimise power consumption until coverage returns, etc.

3)	The AMF indicates to the UE in the Registration Accept whether the UE is not required to perform a Registration procedure when the unavailability period has ended, is delayed or is cancelled.

4)	The AMF may take the Unavailability Period Duration (if available) and Start of Unavailability Period into account when determining Periodic Registration Update timer value. The AMF may provide a Periodic Registration Update time longer than or equal to the combination of the Unavailability Period Duration and Start of Unavailability Period to avoid interfering with the UE dealing with the event that causes the unavailability;

5)	The AMF stores the information that the UE is unavailable at the Start of Unavailability Period in UE context, and considers the UE is unreachable (i.e. clear the PPF in AMF) from then until the UE enters CM-CONNECTED state;

6)	While the UE is unreachable, all high latency communication solutions (see clause 5.31.8) apply if supported in the network, e.g. extended data buffering, downlink data buffering status report, etc; and

7)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period (if available) when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF.

b)	If the UE initiates UE-initiated Deregistration procedure:

1)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF;

Unless the AMF indicated that the UE is not required to perform a Registration procedure when the unavailability period has ended, is delayed or is cancelled, then once the event which makes the UE unavailable is completed in the UE or the event is delayed to a future time or cancelled in the UE, the UE triggers Registration procedure. The UE may also trigger a Registration procedure before the Unavailability Period has started for other reasons. Depending on the UE state, the Registration procedure can be Initial Registration procedure or Mobility Registration Update procedure.

If the UE determines an upcoming loss of coverage no longer applies or determines a new Start of Unavailability Period or Unavailability Period Duration related to the upcoming loss of coverage, the UE sends a new Registration Request.

The UE and the AMF re-negotiate unavailability at every Registration procedure, if it is required. If Start of Unavailability Period and/or Unavailability Period Duration is not included in a Registration procedure any pending unavailability configuration stored in the UE context at AMF is discarded.

Editor's note:	The encoding of the Start of Unavailability Period and the indication for use of Support of Unavailability will be determined by CT WG1.

* * * * End of Changes * * * *