3GPP TSG-SA WG2 Meeting #158	S2-2309127

# Goteborg, Sweden, 21 – 25 August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1348 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the P2A2P procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Regarding establishment of the path in MDC2, - The current P2A procedure first reserves a resource in DCMF, then reserves a resource in DC Application Server, and updates the resource in DCMF. - The current P2A2P procedure first reserves a resource in DC Application Server, then reserves a resource in DCMF, and updates the resource in DC Application Server. This is an unnecessary difference and complicates implementation. Alignment is needed. Considering the direction of SDP offer (from UE#1 to IMS AS) and the direction of information equivalent to SDP offer (from IMS AS via DCSF to DC Application Server), it appears natural to take the current P2A procedure as the basis and to align to it the P2A2P procedure. - The resource in DC Application for UE#2 may need to be updated depending on SDP answer from UE#2. In other words, information equivalent to SDP answer from UE#2 needs to be informed to DC Application Server. But a necessary step is currently missing. - Description in the Figure of step 7 contains a text “DCMF allocates resources for originating MDC2”, which implies the resource is allocated for the originating UE. But the resource needs to be allocated also for the terminating UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Alignment of the P2A2P procedure to the P2A procedure - Step 5 suggests DCSF to access DC Application Server, but DCSF does not need to do so in this step. - Missing steps are added between the current step 9 and 10. - A step is added between the current step 17 and 18 to allow DCSF to inform DC Application Server of information contained in SDP answer from UE#2. - Description in the Figure of step 7 now contains a text “DCMF allocates resources for originating and terminating MDC2”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### AC.7.2.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

The P2A2P procedure requires the establishment of application data channels from UE#1 to DCMF and from UE#2 to DCMF. The two application data channels are associated with the same application. This enables UE#1 and UE#2 to interact with the same DC Application Server simultaneously and the DC Application Server to correlate the data exchanged with both UEs. The P2A procedures as described in clause AC.7.2 are used to establish one application data channel between UE#1 and DCMF and, for A2P scenario, one application data channel between UE#2 and DCMF. In case of two involved UEs, this enables independent communication between UE#1 and UE#2 with the DC Application Server.

Figure AC.7.2.3-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Selected data channel applications are downloaded to UE#1 and possibly UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel, the application data channel information and associated DC application binding information.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters (i.e. associated DC application binding information) in the notification from DCSF and/or operator policies.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the DCMF or MRF.

6.	DCSF invokes Nimsas_MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the DCMF via DC2 or at the MRF via Mr'/Cr based on the DC media information received from DCSF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	DCSF communicates with the DC Application Server for DC resource control and provides information on data channel media resources reserved at the DCMF for both UE#1and UE#2.

10.	DC Application Server accepts the P2A2P application data channel establishment request, returning information equivalent to an MDC2 SDP answer for UE#1 and information equivalent to an MDC2 SDP offer for UE#2.

NOTE 1:	Details on how DCSF communicates with the DC Application Server is out of scope of this Release.

11.	DCSF requests IMS AS to update the DCMF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the DCMF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	The DCSF stores the media resource information and replies to the Nimsas notification request.

15-16.	IMS AS sends re-INVITE which include the SDP offer from DCMF or MRF for the application data channel to the originating S-CSCF and then to the remote network and UE#2.

17-20.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK. Based on the received DC Application binding information in the re-INVITE, UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 2:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

21.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

22.	DCSF notifies the DC Application Server of information deduced from SDP answer from UE#2.

23.	DCSF replies to IMS AS. This reply contains information to be used to derive SDP answer to UE#1, unless DCSF sent the information to IMS AS in step 14.

24-25.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

26.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

27.	P-CSCF returns the 200 OK to UE#1.

28.	UE#1 sends ACK to the terminating network.

29.	The application data channel between UE#1 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

30.	The application data channel between UE#2 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

## ---End of the Change---