3GPP TSG SA WG2 Meeting #158	S2-2309140

# Goteborg, Sweden, August 21 – 25, 2023								(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1352 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of EN for MF registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause AC.7.4.2, the following editor’s note exists: - Editor's note: Usage of IP address and IP address ranges is FFS. To remove this editor’s note, what IP address or IP address range are used needs clarification. It is previously proposed to use IP address or IP address range of P-CSCF, but the proposal was rejected. Therefore it is proposed to clarify that UE IP address and IP address range are used for selecting local MF. Based on IP range planning, the IP address or IP address range of the UE, which are managed by SMF or UPF close to MF, can be configured in the MF. The MF can register the IP address or IP address range to the NRF. The IMS AS can use UE IP address to find a MF close to the UE via NRF by matching the IP address or IP address range the MF registers. This contribution also aligns the usage of term MF by replacing term DCMF to MF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify UE IP address and IP address range are used for MF registration and discovery. - Replace term DCMF to MF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### AC.7.4.2	MF Registration and Discovery

The MF profile includes NF type=MF and may include information to select a local MF to minimize transmission delays in the media path, this may include UE IP address or IP address range which indicates the UE is close to the MF, or location information of the MF.

Editor's note:	Usage of IP address and IP address ranges is FFS.

The MF may update and deregister in the NRF after registration as specified in clauses 4.17.2 and 4.17.3 of TS 23.502 [3].

After registration in the NRF, the MF and its services can be discovered by other consumer NFs as specified in clauses 4.17.4 and 4.17.5 of TS 23.502 [94].

**** End of Changes ****