3GPP SA WG2 Meeting #158	S2-2309209

# Goteborg, Sweden,  August 21 - August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4872 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Grouping and Handling of Traffic from UEs that host PIN Management Clients and PIN Gateway Clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The restriction that a PIN ID can only be used in URSP Rules of a PEGC prevents the PIN ID from being used in URSP Rules of other types of UEs that are part of a PIN (e.g. a UE that hosts a PIN Management Client). The restriction that only PEGCs can be grouped prevents the PEGCs from being grouped with other types of UEs that are part of a PIN (e.g., a UE that hosts a PIN Management Client). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Made an update in the “UE policy delivery for PIN” section so that a PIN ID can be used in the traffic descriptor of a UE instead of only a PEGC. Made an update to the “Session management related policy control” section for PIN so that the policy control can apply to any UE that is part of the PIN. Made updates in the “Identifiers for PIN” section so that any UE can be part of the group that is identified by the External Group ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE that hosts a PIN Gateway Client would need to be configured with multiple URSP Rules for the same PIN and each URSP Rule would need to be configured with different traffic descriptors. One traffic descriptor would be the PIN ID and it is not clear what traffic descriptor would be used in the URSP Rule that handles that PIN Gateway Client traffic. URSP Guidance and QoS request for UEs that are part of a PIN but are not a PEGC would need to be handled separate from the URSP Guidance and QoS request for PEGCs that are part of the same PIN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.2, 5.44.3.2, 5.44.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

### 5.44.2	UE policy delivery for PIN

For a UE registered in the 5GS, the 5GS supports the provisioning of URSP rules that include a PIN ID as Traffic Descriptor. URSP rules with a PIN ID in the Traffic Descriptor are sent to the UE based on the information provided from an AF for PIN as specified in TS 23.502 [3] and TS 23.503 [45] for policy delivery.

**************************** Next Change *****************************

#### 5.44.3.2	Session management related policy control

For PIN traffic via a PDU session, the 5GC supports the policy control. The policy control is based on session management procedures as specified in TS 23.502 [3] and TS 23.503 [45].

An AF may provide QoS parameters for PIN traffic to 5GC as specified in clauses 4.15.6.6, 4.15.6.6a, and 4.15.6.14 of TS 23.502 [3]. The PIN traffic can be categorised into following types:

-	PIN indirect communication.

-	PIN-DN communication.

An AF may influence traffic routing for PDU sessions for PIN-DN communication as specified in clause 5.6.7 and in clause 4.3.6 of TS 23.502 [3].

**************************** Next Change *****************************

### 5.44.4	Identifiers for PIN

A PIN is managed at the PIN application layer. In 5GS a PIN ID is only used in the traffic descriptor of URSP rules, for routing traffic of specific PIN towards a dedicated (DNN, S-NSSAI) combination. If a PIN contains more than one UE (e.g., more than one PEGC), the list of UEs may be grouped together following the 5G VN group management principles as specified in clause 5.29.2. Then the UEs of a PIN can be identified by an External Group ID by the AF for PIN. The AF for PIN may use the External Group ID to manage the list of UEs that are part of a PIN and for providing URSP guidance and/or QoS requests applicable to all the UEs.

*************************** End of Changes ****************************