3GPP TSG-WG SA2 Meeting #158	S2-2309230

# Goteborg, Sweden 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0182 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter V-SMF Idle mode mobility in HR-SBO case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure needs a figure and more precise description for the Inter V-SMF idle mode mobility in HR-SBO case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a figure of the procedure and clarify its description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Idle mode mobility proceldure is missing for HR-SBO PDU Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Changes * * * *

6.7.2.7	Inter V-SMF (intra-VPLMN and inter-PLMN) Idle mode mobility in HR-SBO case

This clause defines the procedure for inter V-SMF intra-VPLMN and inter-PLMN Idle mode mobility for HR-SBO PDU Sessions. This procedure is based on Idle mode mobility with I-SMF insertion/change/removal defined in clause 4.23 of TS 23.502 [3].

Figure 6.7.2.7-1: Inter V-SMF idle mobility in HR-SBO case

1.	The UE establishes a HR-SBO PDU Session described in clause 6.7.2.2.

2.	The UE initiates Mobility Registration Update procedure to the Target AMF as described in clause 4.2.2.2.2 of TS 23.502 [3]. During the registration procedure, the target AMF (in roaming case) receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSSAI from the UDM in the step 14b of the procedure in clause 4.2.2.2.2 of TS 23.502 [3].

The DNN/S-NSSAI of the established PDU Session with HR-SBO in the Source PLMN may not allowed for HR-SBO in the Target PLMN. For this case, the H-SMF revoke the HR-SBO authorisation for the established PDU Session with DNN/S-NSSAI. For inter-PLMN scenario, the AMF shall trigger the V-SMF insertion or V-SMF change procedure by invoking Nsmf_PDUSession_SMContext Create.

3.	(V-SMF insertion or V-SMF change) the Target AMF selects a V-SMF and sends the received HR-SBO allowed indication for the established Session for DNN, S-NSSAI in the PDUSession_SMContext Create Request to the target V-SMF.

4.	As in step 4 of Figure 6.7.2.6 -1.

5.	As in step 5 of Figure 6.7.2.6 -1.

6.	The target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF as in step 7 of Figure 6.7.2.6-1.

7.	As in step 8 of Figure 6.7.2.6 -1.

8.	As in step 9 of Figure 6.7.2.6 -1, The H-SMF sends the VPLMN Specific Authorization Information in the Nsmf_PDUSession_Update Response message to the V-SMF.

9.	As in step 9 of Figure 6.7.2.6 -1, the V-SMF may configure the V-EASDF, V-UPF based on the received VPLMN Specific Authorization Information.

10.	The target V-SMF answers to the target AMF and the registration procedure continues as defined in clause 4.2.2.2.2 of TS 23.502 [3].

# * * * * Next Changes * * * *

# * * * * End Changes * * * *