3GPP SA WG2 Meeting #158	S2-2309370

# Goteborg, Sweden,  August 21 - August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4915 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN on ESNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a following EN in the spec: Editor's note: Whether the Equivalent SNPN(s) has same validity information as the SNPN that providing access for Localized Services the UE was last registered is for FFS We see below approaches to solve this: - Localized services supported by RSNPN are also also supported in equivalent SNPN thus UE can consider eSNPN during SNPN selection. - If eSNPN validity information is configured(following existing mechanism for any other SNPN), then UE can check if those are met then UE can consider eSNPN during SNPN selection. - “a” is applied only if eSNPN validity information is not configured. If configured then “b” is applied. In this release we believe option “a” has least impact thus the same is proposed in this CR. We can live with option-b or option-c too if group prefers then this CR can be aligned accordingly in the revised version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE can select eSNPN because in this release, equivalent SNPN(s) are assumed to provide the same localised services as the last registered SNPN provides |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN will remain unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** Start of Changes ***************************

##### 5.30.2.4.2	Automatic network selection

NOTE 1:	If the UE has multiple subscriptions (SNPN and/or PLMN) it is assumed that the subscription to use for automatic selection is determined by implementation specific means prior to network selection.

If the UE supports accessing an SNPN providing access for Localized Services and the end user enables to access Localized Services, for automatic network selection, the UE shall select and attempts registration on available SNPN in the following order:

(a)	if the UE supports access to an SNPN using Credentials from a Credentials Holder then the UE continues by selecting and attempting registration on available and allowable SNPNs which broadcasts the indication that access using credentials from a Credentials Holder is supported in the following order:

i	the SNPN with the validity information the UE was last registered with (if the validity information is met) or the equivalent SNPN (if available and the validity information of last registered SNPN is met);

NOTE 2:	In this release, equivalent SNPN(s) are assumed to provide the same localised services as the last registered SNPN provides.

ii	SNPNs in the Credentials Holder controlled prioritized list of preferred SNPNs for accessing Localized Services (in priority order) if the validity information is available and is met;

iii	SNPNs, which additionally broadcast a GIN contained in the Credentials Holder controlled prioritized list of preferred GINs for accessing Localized Services (in priority order) if validity information is available and is met;

(b)	the SNPN without validity information the UE was last registered with (if available) or the equivalent SNPN (if available);

(c)	the subscribed SNPN, which is identified by the PLMN ID and NID for which the UE has SUPI and credentials;

(d)	the available and allowable SNPNs which broadcast the indication that access using credentials from a Credentials Holder is supported in the following order:

i	SNPNs in the user controlled prioritized list of preferred SNPNs (in priority order);

ii	SNPNs in the Credentials Holder controlled prioritized list of preferred SNPNs (in priority order);

iii	SNPNs, which additionally broadcast a GIN contained in the Credentials Holder controlled prioritized list of preferred GINs (in priority order);

iv-	SNPNs, which additionally broadcast an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e. the broadcasted NID or GIN is not present in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs, nor in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs for accessing Localized Services in the UE.

If the UE supports accessing an SNPN providing access for Localized Services and the end user enables to access Localized Services, the UE shall periodically attempt reselection and registration on a higher priority SNPN 1) based on the order of the above sub-bullets (i) to (iii) of bullet (a), bullet (c), sub-bullets (i) to (iii) of bullet (d) if the UE is not registered to the sub-bullet (i) of bullet (a) or 2) based on the order of the above sub-bullets (ii) to (iii) of bullet (a), bullet (c), sub-bullets (i) to (iii) of bullet (d) if the UE is registered to the sub-bullet (i) of bullet (a) if any of the below conditions is met:

-	if there are one or more SNPNs with validity information which is met, and the UE is not registered to an SNPN which has highest priority among the one or more SNPNs; or

-	if there is no SNPN with validity information which is met, and there are one or more GINs with the validity information which is met, and the UE is not registered to an SNPN broadcasting a GIN which has highest priority among the one or more GINs; or

-	if there is no SNPN with validity information which is met and there is no GIN with validity information which is met, and the UE is not registered to the subscribed SNPN

Otherwise, the UE does not trigger periodic reselection and does not attempt registration on a higher priority SNPN

NOTE 3:	Details of network selection (e.g. validity information handling, periodicity determination) specified in TS 23.122 [17].

If a validity condition in Credentials Holder controlled prioritized lists of preferred SNPNs/GINs for accessing Localized Services changes from met to not met (and vice versa), the UE shall attempt selection and registration on an SNPN based on the above bullets (a) to (d).

If the UE does not support to access an SNPN providing access for Localized Services or the end user does not enable to access the Localized Services, for automatic network selection the UE shall select and attempts registration on available and allowable SNPNs in the following order:

-	the SNPN without validity information the UE was last registered with (if available) or the equivalent SNPN (if available);

-	the subscribed SNPN, which is identified by the PLMN ID and NID for which the UE has SUPI and credentials.;

-	If the UEs supports access to an SNPN using credentials from a Credentials Holder then the UE continues by selecting and attempting registration on available and allowable SNPNs which broadcast the indication that access using credentials from a Credentials Holder is supported in the following order:

-	SNPNs in the user controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs in the Credentials Holder controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs, which additionally broadcast a GIN contained in the Credentials Holder controlled prioritized list of preferred GINs (in priority order);

NOTE 4:	If multiple SNPNs are available that broadcast the same GIN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

-	SNPNs, which additionally broadcast an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e. the broadcasted NID or GIN is not present in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs in the UE.

NOTE 5:	If multiple SNPNs are available that broadcast the indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

When a UE performs Registration or Service Request to an SNPN, the UE shall indicate the PLMN ID and NID as broadcast by the selected SNPN to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

*************************** End of Changes ****************************