3GPP SA WG2 Meeting #158	S2-2309402

Gothenburg, Aug 21 – 25 2023	(revision of S2-22xxxx)

# Source:	Vodafone, Deutsche Telekom, BT, Huawei, Xiaomi, InterDigital

# Title:	Draft TEI19 WID on support of Access Traffic Steering, Switching, Splitting and Aggregation of all Traffic Types by single MPDCCP based Steering functionality

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Support of Access Traffic Steering, Switching, Splitting and Aggregation of all Traffic Types by single MPDCCP based Steering functionality

Acronym:	TEI19_ATSSS_MPDCCP

Unique identifier:	tbd

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ATSSS_Ph3 | SA2 | 940070 | Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture Phase 3 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Up to Rel. 18, Splitting and aggregation support for TCP and UDP traffic is specified relying on solutions based on MP-TCP and MP-QUIC respectively. Although the Steering and Switching capabilities for IP and Ethernet traffic are supported by the means of the ATSSS-LL functionality, the ATSSS framework still lacks a solution capable of providing splitting and aggregation support for these traffic types. Different industrial applications as well as the 5G-RG scenario, would benefit of such function, as it would enable the possibility of having traffic aggregation or applying redundant steering without being limited by the transport protocol in place (TCP/UDP).

A single steering functionality, based on MPDCCP, as studied in release 18 in TR 23.700-53 section 6.3 Solution #2.1: MP-DCCP based Steering Functionality”,

- supports all kind of traffic splitting and aggregation;

- applications can enjoy multi-path transmission without the need to being aware of it;

- no need to configure for which traffic to use which steering functionality and which proxy;

- no need to implement a separate proxy for each traffic type;

- does not require double encryption;

- MPDCCP is on standards track in IETF now.

While in release 18 due to limited available time units only one new steering functionality could go forward for normative phase, this limitation may not apply in this release, and the request from BBF in LS S2-2205467 to consider MPDCCP can be fulfilled.

# 4	Objective

The objective of this TEI work item is to introduce a new steering functionaltiy that supports all kind of traffic splitting and aggregation, which will be based on “Solution #2.1: MP-DCCP based Steering Functionality” in TR 23.700-53 to the ATSSS architecture into the normative specifications.

The specified steering functionality shall be able to support all ATSSS steering modes (i.e. the redundant steering mode and all other steering modes defined in previous releases).

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | System Architecture for the 5G System; Stage 2 Adding MPDCCP steering functionality to the architecture reference model for ATSSS and update ATSSS section accordingly. | SA#99 (March 2023) |  |
| 23.502 | Procedures for the 5G System; Stage 2 Update the ATSSS procedures to support the new MPDCCP steering functionality. | SA#99 (March 2023) |  |
| 23.503 | Policy and Charging Control Framework for the 5G System; Stage 2 Update the session management-related policy control for ATSSS to support the new MPDCCP steering functionality. | SA#99 (March 2023) |  |

This TEI19_WID requires 1 TU.

# 6	Work item Rapporteur(s)

Vodafone, Susana Sabater

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None identified.

# 9	Supporting Individual Members

Supporting IM name

Vodafone

Deutsche Telekom

Xiaomi

Huawei

BT

InterDigital