3GPP TSG-SA WG2 Meeting #158	S2-2309494

# Goteborg, Sweden, August 21 – 25, 2023								  (revision of S2-2308839)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0019 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Ranging/SL Positioning Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The content of Ranging/SL Positioning Capability is not clearly described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the description of Ranging/SL Positioning Capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description of Ranging/SL Positioning Capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 4.3.1, 4.3.2, 4.3.3, 5.1.2, 6.3.2, 6.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

4.2.4	Reference points

SR1:	The reference point between the Sidelink (SL) Positioning and Ranging function in the UE and the SL Positioning/Ranging Server. This reference point is out of scope of this specification. It may be used for the configuration and application layer signalling.

SR5:	The reference point between the Sidelink (SL) Positioning and Ranging function in UEs. It is carried over PC5-U of the PC5 reference point.

PC5:	The reference point between the UEs. It also supports the Sidelink Positioning and Ranging operation defined by RAN WGs.

N1:	In addition to the relevant functions defined in TS 23.501 [2] for N1, in the case of Ranging/SL Positioning Service, it is also used to convey the Ranging/SL Positioning policy and parameters (including service authorization) from AMF to UE, and to convey the UE's Ranging/SL Positioning Capability from UE to AMF.

N2:	In addition to the relevant functions defined in TS 23.501 [2] for N2, in the case of Ranging/SL Positioning Service it is also used to convey the Ranging/SL Positioning policy and parameters (including service authorization) from AMF to NG-RAN.

* * * * 2nd Change * * * *

4.3.1	UE

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UE may support the following functions:

-	Reporting the following Ranging/SL Positioning Capabilities to 5GC over the N1 reference point:

-	Capability of supporting Ranging/SL Positioning over PC5;

NOTE:	Based on Ranging/SL Positioning control, a UE capable of Ranging/SL Positioning may take different roles in the operation, e.g. Target UE, Reference UE, Located UE.

-	Capability of supporting SL Positioning Server UE over PC5.

-	Procedures for Ranging/SL Positioning over PC5.

-	Procedures to Network based SL Positioning, and Network assisted SL Positioning;

-	Procedures to Ranging/SL Positioning service exposure.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered Ranging/SL Positioning Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for Ranging/SL Positioning over PC5;

-	Policy/parameters for Located UE;

-	Policy/parameters for Target UE in addition to the functions defined in TS 23.273 [8] clause 4.3.5;

-	Policy/parameters for SL Positioning Client UE;

-	Policy/parameters for SL Positioning Server UE.

-	Receiving the Ranging/SL Positioning Policy from 5GC over N1 reference point.

-	Configuration of parameters for Ranging/SL Positioning over PC5. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over SR1 reference point from the Ranging/SL Positioning Application Server.

* * * * 3rd Change * * * *

4.3.2	PCF

In addition to the functions defined in TS 23.501 [2] and TS 23.503 [4], the PCF includes functions to provision the UE with necessary policies and parameters to use Ranging/SL Positioning services, as part of the UE Ranging/SL Positioning Policy information as defined in TS 23.503 [4] clause 4.2.2:

-	Authorization policy and parameters for Ranging/SL Positioning over PC5;

-	Authorization policy and parameters for Network based SL Positioning, and Network assisted SL Positioning;

-	Authorization policy and parameters for Ranging/SL Positioning service exposure.

The PCF may update the Ranging/SL Positioning to the UE under certain conditions.

When receiving the Ranging/SL Positioning Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters for RSPP transport over PC5 reference point used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [6].

* * * * 4th Change * * * *

4.3.3	AMF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL Positioning Policy/Parameter provisioning based on indication of Ranging/SL Positioning Capability as part of the "5GMM capability" in the Registration Request.

-	Store the Ranging/SL Positioning Capability.

-	Select the LMF supporting Ranging/SL positioning and forward the Ranging/SL Positioning Capability of Target UE to LMF.

-	Select a GMLC supporting Ranging/SL positioning and request the GMLC for Ranging/SL Positioning result.

-	Forward the Ranging/SL Positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5 (i.e. as Ranging/SL positioning-enabled UE).

-	Provision the NG-RAN with PC5 QoS parameters related to RSPP transport over PC5.

* * * * 5th Change * * * *

5.1.2	Authorisation and policy/parameter provisioning to NG-RAN

The "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided to the NG-RAN node for scheduled resource allocation mode resource management.

The "Ranging/Sidelink Positioning authorised" information includes one or more of the following:

-	whether the UE is authorized to act as a Reference UE;

-	whether the UE is authorized to act as a Target UE;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

The RSPP transport QoS parameters include parameters defined in clause 5.7.3, which are generated by the PCF and are provided to the AMF when receiving the Ranging/Sidelink Positioning Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR.

During registration, the UE includes the Ranging/Sidelink Positioning Capability as defined in clause 4.3.1 as part of the "5GMM capability" in the Registration Request message. AMF determines whether the UE is authorised to use Ranging based services and Sidelink positioning based on UE's Ranging/Sidelink Positioning Capability and the Ranging/Sidelink Positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the Ranging/Sidelink Positioning Capability in the UE context.

If the UE is authorised to use Ranging based services and Sidelink positioning, the "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided from the AMF to the NG-RAN node in a NGAP message during Registration procedure, Service Request procedure and N2 Handover procedure, and from source NG-RAN node to target NG-RAN node over Xn during Xn Handover procedure as defined in clause 6.5 of TS 23.287 [6] and clause 6.6 of TS 23.304 [7].

* * * * 6th Change * * * *

6.3.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [3] clause 4.2.2.2 with the following additions:

-	The UE includes the Ranging/SL Positioning Capability as defined in clause 4.3.1 as part of the "5GMM capability" in the Registration Request message. The AMF stores the Ranging/SL Positioning Capability for Ranging/SL Positioning operation.

NOTE:	For 5G ProSe capable UE or V2X capable UE, the 5G ProSe Capability as defined in TS 23.304 [7] or the V2X Capability as defined in TS 23.287 [6] is included along with Ranging/SL Positioning Capability in the "5GMM capability".

-	The AMF obtains the Ranging/SL positioning subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [3].

-	The AMF determines whether the UE is authorised to use Ranging/SL positioning services based on UE's Ranging/SL Positioning Capability and the Ranging/SL positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the authorized Ranging/SL Positioning Capability.

-	The AMF sends the authorized Ranging/SL Positioning Capability for Ranging/SL positioning operation to PCF. Based on the received Ranging/SL Positioning Capability from the AMF, the PCF provides the PC5 QoS parameters related to Ranging/SL positioning to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"Ranging/SL positioning services authorised" information, including one or more of the following:

-	whether the UE is authorized to use Ranging/SL Positioning over PC5;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

-	the PC5 QoS parameters related to Ranging/SL positioning used by the NG-RAN for the resource management of UE's PC5 transmission for Ranging/SL Positioning services in network scheduled mode.

Editor's note:	"Ranging/SL positioning services authorised" information needs the coordination with RAN WGs.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [3].

* * * * 7th Change * * * *

6.3.7	Delivery of PC5 QoS parameters related to Ranging/SL positioning to NG-RAN

The UE Policy Association Establishment procedure and UE Policy Association Modification procedure, as defined in TS 23.502 [3], is used to provide the AMF with PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN. When receiving the Ranging/SL Positioning Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN corresponding to a UE.

The (V-)PCF provides the information to the AMF as follows:

-	In the roaming case, the H-PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF receives the N2 PC5 policy container from (V-)PCF, the AMF stores such information as part of the UE context.

In the UE Configuration Update procedure triggered by UE Policy Association Establishment or UE Policy Association Modification:

-	The AMF forwards the PC5 QoS parameters related to Ranging/SL positioning in the NGAP message to the NG-RAN if a N2 PC5 policy container is received in the Namf_Communication_N1N2MessageTransfer message.

NOTE 1:	If the PC5 QoS parameters related to Ranging/SL positioning are provided to both NG-RAN and UE, both the N2 PC5 Policy Container and the UE Policy Container are included in the Namf_Communication_N1N2MessageTransfer message.

NOTE 2:	Non-UE specific PC5 QoS parameters, e.g. default PC5 QoS parameters, can also be locally configured in NG-RAN. How such configuration is performed is out of scope of this specification.

* * * * End of Changes * * * *