3GPP TSG-WG SA2 Meeting #158	S2-2309580

# 21 - 25 August, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4256 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct PFD management based on PFD Determination analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.18.2.2 not complete PFD information related descriptions will arouse wrong perception, step1 missing Nnwdaf_AnalyticsInfo_Response service operation, step 2 may be optional if the PFD data is already available in NEF (PFDF) and step 4 may also be optional if the NEF(PFDF) determine not to create/update/delete PFD information, that is dot line figure should be used in step 2 and step 4 and correspding procedure description should be updated to be clear. And step 4 clause 4.18.2 needs to be corrected as clause 4.18.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 4.18.2.2 correct "PFD information subscribed from the NWDAF" as PFD information notified/responded from the NWDAF, also correct the figure and procedure description in step1, step 2 and step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and not complete NEF(PFDF) handling on PFD determination analytics will arouse wrong stage 3 implementation and wrong deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.18.2.1,4.18.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.18.2.1	PFD management triggered by AF

Figure 4.18.2.1-1 procedure for PFD management via NEF (PFDF) triggered by AF

1.	The AF invokes the Nnef_PFDManagement_Create/Update/Delete service. The Allowed Delay is an optional parameter. If the Allowed Delay is included, it indicates that the list of PFDs in this request should be provisioned within the time interval indicated by the Allowed Delay to the SMF(s) that have subscribed to the PFD management service using Nnef_PFDManagement_Subscribe service operation.

2.	NEF checks whether the AF is authorized to perform this request and NEF (PFDF) checks if the AF is authorised to provision this PFD data based on the operator policies. Besides the information from the AF, the NEF may subscribe to the NWDAF to receive PFD Determination analytics defined in clause 6.16.3 of TS 23.288 [50].

3.	The NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs, Allowed Delay) to the UDR.

4.	The UDR updates the list of PFDs for the Application Identifier.

5.	The UDR sends a Nudr_DM_Create/Update/Delete Response to the NEF (PFDF).

6.	The NEF sends Nnef_PFDManagement_Create/Update/Delete Response to the Application Function.

#### 4.18.2.2	PFD management based on PFD Determination analytics

Figure 4.18.2.2-1 shows the procedure that NEF (PFDF) determines the PFD information for the known Application Identifier(s), based on the PFD Determination analytics information notified/responded from the subscribed/requested NWDAF. The procedure enables the NEF(PFDF) to determine whether to create/update/delete PFD information corresponding to the known Application Identifier(s).

Figure 4.18.2.2-1: Procedure for PFD management based on PFD Determination analytics

1.	The NWDAF notifies/responds to PFD Determination analytics to the NEF (PFDF) as Consumer NF with PFD Information defined in clause 6.16.3 of TS 23.288 [50].

2.	The NEF (PFDF) fetches the PFD information currently in use from UDR if not available in NEF (PFDF), as described from step 2 to step 3 of clause 4.18.3.1.

3.	The NEF (PFDF) compares the PFD information from UDR with PFD information from the NWDAF to determine whether to create/update/delete PFD information corresponding to the Application Identifier.

4.	If the NEF (PFDF) has determined in step 3 to create/update/delete PFD information corresponding to the Application Identifier, the NEF (PFDF) invokes the Nudr_DM_Create/Update/Delete (Application Identifier, one or more sets of PFDs) to the UDR to create/update/delete PFD information corresponding to the Application Identifier, i.e. from step 3 to step 5 of clause 4.18.2.1 are executed..

*** End of Changes ***