SA WG2 Meeting #158	S2-2309746

21 - 25 August, 2023, Goteborg, Sweden	revision of S2-2308462

Title:	Reply LS on MBS service area update clarification

Response to:	LS on MBS service area update clarification (S2-2308285 /S6-231606)

Release:	Rel-17

Work Item:	5MBS

Source:	3GPP TSG SA WG2

To:	3GPP TSG SA WG6

CC:	3GPP TSG CT WG3, 3GPP TSG CT WG4

Contact person:	Thomas Belling

Thomas (dot) belling (at) Nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

SA2 thanks SA6 for their LS, where SA6 wrote:

In public safety and verticals scenarios, the service area may be changed due to the mobility of the group members. SA6 has concerns that failure may occur once the AF triggers the MBS session update with a new MBS service area. …

Q1. SA6 would like to ask SA2 whether the AF/NEF will interact with the NRF to re-select the capable MB-SMF during the MBS session update with requested MBS service area?

Q2. SA6 would like to ask CT3 and CT4 to clarify the scenario(s) when the error “UNKNOWN_MBS_SERVICE_AREA 404 Not Found The requested MBS service area (e.g. identified by the Area Session ID) cannot be found.” in 3GPP TS 29.522 can occur? Is the MBS service area not found in MB-SMF, NRF or NEF? Can it occur during MBS session update?

Q3. SA6 would like to ask SA2 to confirm whether the scenarios described above from SA6, CT4 and CT3 can occur during MBS session update. If yes, what are the mechanism(s) defined by SA2 to be employed to overcome an MBS session update failure?

SA2 would like to answer the questions as follows:

Q1:

There will not be any MB-SMF re-selection during MBS session update.

Q3:

SA2 regards the scenario outlined by SA6 as an error scenario:

In 5MBS, an MBS service area in an MBS session create or update request is assumed to be within the same MB-SMF service area of a single MB-SMF.

If the AF requests an unacceptable service area (not entirely contained in a single MB-SMF service area), the session creation or update request may be rejected by the 5GC or may be accepted by the 5GC to handle only the acceptable part of the MBS service area (depending on further stage 2 discussion). It is expected that appropriate information about the acceptable or rejected parts of the service area will be defined and provided in the response to the AF in such situations.

SA2 intends to further refine the stage 2 TS 23.247 in future meetings to clarify this matter.

# 2	Actions

To SA6

ACTION: 	SA2 asks SA6 to kindly take the provided feedback into account.

# 3	Dates of next TSG SA WG 2 meetings

SA2#159                 09th -13th October 2023 	Xiamen, CN