SA WG2 Meeting #158	S2-2309750

# 21-25 Aug 2023, Goteborg, Sweden	(was S2-2309297)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4433 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small Data Transmission for Redcap UE in RRC inactive with eDRX>10.24s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In last SA2 meeting the connection resume for SDT was introduced in clause 4.8.2.2. The text is ambiguity and some clarifications are needed |  |  |  |  |  |  |  |  |
| t |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on connection resume for SDT handling in clause 4.8.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is ambiguity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

4.8.2.2	UE Triggered Connection Resume in RRC_INACTIVE procedure

The Connection Resume procedure is used by the UE in RRC_INACTIVE state, e.g. to transition to RRC_CONNECTED state or for Small Data Transmission while in RRC_INACTIVE as specified in TS 38.300 [9]. Triggers for the UE to initiate this procedure are defined in clause 5.3.3.2.5 of TS 23.501 [2].

Figure 4.8.2.2-1: Connection Resume in RRC_INACTIVE

1.	UE to NG-RAN: RRC message (Resume ID).

The UE initiates connection resume from RRC_INACTIVE state, see TS 38.300 [9]. The UE provides its Resume ID needed by the NG-RAN to access the UE's stored Context.

2.	[Conditional] NG-RAN performs UE Context Retrieval.

UE Context Retrieval is performed when the UE Context associated with the UE attempting to resume its connection is not locally available at the accessed NG-RAN. The UE Context Retrieval procedure via NG-RAN is specified in TS 38.300 [9].

3.	NG-RAN to UE: RRC messages.

NG-RAN determines whether the UE shall be transitioned to RRC_CONNECTED state or kept in RRC_INACTIVE (e.g. the latter in the case of Small Data Transmission as defined in TS 38.300 [9]).

4a.	[Conditional] N2 Path switch procedure.

If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding.

If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2.

4b.	[Conditional] N2 Notification,

4b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC_INACTIVE and still has the UE context, NG-RAN sends:

-	an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3); or

-	an MT Communication Handling request to the AMF indicating the UE is now reachable for downlink data and/or signalling if Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

4b.2	The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.

4b.3	N4 session modification procedure is triggered by the SMF. If data buffering is handled in the UPF, the SMF updates the UPF with appropriate rules to trigger data delivery.

4b.4	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

4b.5	The AMF sends the N2 MT Communication Handling response message to NG-RAN.

If NG-RAN determines that the connection resume is for Small Data Transmission as defined in TS 38.300 [9] and step 4a or steps 4b.1 to 4b.5 have been performed, then NG-RAN keeps the UE in RRC_INACTIVE state and the UL/DL Small Data are transferred via the NG-RAN. Based on the procedures defined in TS 38.300 [9],if the UE is re-configured with RRC Inactive with eDRX>10.24s, the NG-RAN may send an N2 message to 5GC as described in step 2 in clause 4.8.1.1a so the CN can then handle mobile terminated (MT) communication.

# >>>>END OF CHANGES<<<<