3GPP TSG-WG SA2 Meeting #158	S2-2309795

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S2-2308980)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0172 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on procedure of Handling of Common EAS, CommonDNAI for set of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some clarification on the procedure of Handling of Common EAS, CommonDNAI for set of UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | clarification on the procedure of Handling of Common EAS, CommonDNAI for set of UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding about the procedure may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.2.7	Coordination among SMFs for Common EAS/DNAI determination

Figure 6.2.3.2.7-1: Handling of Common EAS, Common/DNAI for set of UEs

1.	SMF sends Nsmf_TrafficCorrelation_Notify to the NEF with Notification Endpoint received in the PCC rule as described in clauses 6.2.3.2.5 and 6.2.3.2.6 and provides: EAS IP address based on EASDF procedure, list of candidate DNAI(s), SMF ID, number of PDU sessions it is serving for the set of UEs.

2.	If the NEF determines that there is currently no common EAS IP address and/or common DNAI available for the set of UEs identified by Traffic Correlation ID, it selects a common DNAI and/or common EAS using the list of DNAI(s) and/or EAS IP address received in step 1. Then the NEF updates traffic influence data with the 5GC determined common EAS/DNAI for the set of UEs.

The update of traffic influence data triggers notifications to PCF(s) that in turn trigger associated PCC rule updates to the SMF(s), if any, with PDU Session(s) associated with the traffic correlation ID.

The NEF maintains a list of SMFs serving the set of UEs and the associated data received in step 1.

3.	NEF responds by acknowledging to the SMF.

4.	The update in UDR triggers notification to the PCF(s) that have subscribed for notification. The PCF(s) sends PCC rule(s) with common EAS and/orCommon DNAI and Traffic correlation ID, as part of traffic influence data to the SMF(s) with PDU Session(s) associated with the Traffic Correlation ID.

SMF(s) may select other candidate DNAI(s) for the PDU session(s) or a candidate new EAS IP address via the EASDF procedure e.g. due to UE(s) mobility. In this case, the SMF notifies to the NEF as in the above step 1, with the list of candidate DNAI(s)  and/or EAS IP address. This may trigger NEF to re-select common DNAI or common EAS. NEF determines common EAS and/or common DNAI based on received EAS IP, list of candidate DNAI(s), number of PDU sessions SMF(s)  serving for the set of UEs.

If another DNAI/EAS IP address is selected by the NEF, it updates the common DNAI or common EAS in the UDR in the Traffic Influence data.

NOTE:	How NEF determines a common EAS/DNAI is implementation.

# * * * * End of changes * * * *