3GPP TSG-WG SA2 Meeting #158	S2-2309815

# Goteborg, Sweden, August 21-25, 2023								 			 revision of S2-2309444

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1140 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XRM corrections in various sections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Section 5.3.1 misses the information required to support XRM services. Section 6.1.3.27.4 misses propagation of information in PCC rules to SMF and subsequently the UPF for proper XRM/PDU Set related handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sections 5.3.1, 6.1.3.27.4 are updatd accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 6.1.3.27.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 5.3.1	Interactions between PCF and AF

Npcf and Naf enable transport of application level session information and Ethernet/IP port management information from AF to PCF. Such information includes, but is not limited to:

-	IP filter information or Ethernet packet filter information to identify the service data flow for policy control and/or differentiated charging;

-	media/application bandwidth requirements for QoS control;

-	In addition, for sponsored data connectivity:

-	the sponsor's identification;

-	optionally, a usage threshold and whether the PCF reports these events to the AF;

-	information identifying the application service provider and application (e.g. SDFs, application identifier, etc.);

-	information required to enable Application Function influence on traffic routing as defined in clause 5.6.7 of TS 23.501 [2];

-	information required to enable Application Function influence on Service Function Chaining as defined in clause 5.6.16 of TS 23.501 [2];

-	information required to enable setting up an AF session with required QoS as defined in clause 6.1.3.22;

-	information required to enable setting up an AF session with support for Time Sensitive Networking (TSN) as defined in clause 6.1.3.23.

-	information required to enable setting up an AF session with support for Time Sensitive Communication as defined in clause 6.1.3.23a.

-	information required to enable setting up an AF session with support for Deterministic Networking (DetNet) as defined in clause 6.1.3.23b.

-	information required to enable setting up an AF session with support for eXtended Reality and Interactive Media Services (XRM) as defined in clause 6.1.3.27.

Npcf also enables the AF to request to influence Access and Mobility related policies for a UE and enables the AF to provide guidance for UE URSP rule determination. Npcf and Naf enable the AF subscription to notifications on PDU Session events, i.e. the events requested by the AF as described in clause 6.1.3.18 and the change of DNAI as defined in clause 5.6.7 of TS 23.501 [2].

The N5 reference point is defined for the interactions between PCF and AF in the reference point representation.

# * * * * 2nd change * * * *

##### 6.1.3.27.4	PDU Set QoS handling

PDU Set QoS handling support is defined in clause 5.37.5 of TS 23.501 [2]. The PCF may, based on local configuration or on information received from the AF, generate PDU Set Control Information in the PCC Rule including the PDU Set QoS Parameters (see clause 5.7.7 of TS 23.501 [2]) and the Protocol Description (see clause 5.37.5 of TS 23.501 [2] and TS 26.522 [40]). Based on the received the PCC rule or a pre-configured PCC rule, the SMF provides NG-RAN the PDU Set QoS Parameters (as described in clause 5.37.5 of TS 23.501 [2]). If the PCC rule includes the Protocol Description, it is provided to the PSA UPF for PDU Set identification (as described in clause 5.37.5 of TS 23.501 [2]).

The PCF generates the PDU Set Control Information within a PCC Rule for the service data flow with PDU Sets based on the information provided by the AF or on local policies. When the AF requests that a data session to a UE is set up with a specific QoS (as described in clause 6.1.3.22), the AF can also provide the individual QoS parameters Requested PDU Set Delay Budget, Requested PDU Set Error Rate and Requested PDU Set Integrated Handling Information to describe the requirements for the PDU Set QoS handling and the Protocol Description.

# * * * * End of change * * * *