SA WG2 Meeting #158	S2-2309860

# 21-25 August 2023, Gothenburg, Sweden	(was S2-2309837)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4907 | rev | 4 | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set QoS handling: Race conditions at mobility from supporting to non-supporting gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, CATT, Ericsson, Nokia, Nokia Shanghai Bell, OPPO, ZTE, Samsung, vivo, China Mobile, Huawei, Lenovo, NTT DoCoMo, Google, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During SA2#157 the issue of handover from a RAN node not supporting to a RAN node supporting the PDU Set QoS handling led to capturing the following editor’s note in clause 5.37.5.3: Editor's note: In the case of handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, whether there is a reason for the target NG-RAN node to temporarily handle both unmarked PDUs and marked PDUs within the same QoS flow is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that based on the NG-RAN indication of PDU Set QoS support, the SMF may initiate the PDU Session modification procedure to provide PDU Set QoS parameters for QoS flow(s) to NG-RAN and may configure the PSA UPF to activate the PDU Set identification and marking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unspecified how to handle PDU Set QoS during mobility beween non-supporting and supporting RAN nodes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set based handling is supported. Based on this, SMF may activate the PDU Set identification and marking in the PSA UPF.

At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indication, the SMF may, upon completion of the handover procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters to NG-RAN and configure the PSA UPF to activate/deactivate the PDU Set identification and marking.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

# >>>>END OF CHANGES<<<<