SA WG2 Meeting #S2-158	S2-2309923

21 - 25 Aug, 2023, Goteborg, SE

# Source:	ZTE

# Title:	Study on UPF enhancement for Exposure and SBA Phase 2

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on UPF enhancement for Exposure And SBA Phase 2

Acronym:		FS_UPEAS_Ph2

Unique identifier:	xxxxxx

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

{Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN}

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 790007 | Study on Enhancements to the Service-Based 5G System Architecture (Release 16) | Study of extending SBA to user plane but without standardization |
| 940076 | Study on UPF enhancement for Exposure And SBA | Corresponding study of architecture enhancements and procedures (SA2) |
| 970019 | UPF enhancement for Exposure and SBA | Rel-18 WI for Enhancement of support for UPF Event Exposure and SBA (SA2) |
| 990002 | CT aspects of UPF enhancement for exposure and SBA | Impacts on protocols and interfaces to support the UPF enhancement for Exposure and SBA (CT4) |

3	Justification

UPF Event Exposure capability is supported in 5GS since Rel-17. In Rel-18 FS_UPEAS study, further enhancements for supporting UPF enhancement for Exposure and SBA are studied, including UPF event exposure service(s) registration/deregistration and discovery via the NRF, UPF event exposure services enhancement etc. Two key issues from FS_UPEAS study have been concluded and progressed to the normative phase according to TR 23.700-62.

During Rel-18 study, some other issues were raised, however not studied due to the time limitation. The issues include:

-	Enhancing UPF and other NF event exposure to support additional exposure information to/from NWDAF, AF/NEF and other NFs. This issue is derived from the requirements of services such as AIML, XRM, and sensing.

-	Supporting modular deployment of user plane, which can enable the functionalities to be dynamically and flexibly inserted in the data path of a PDU session according to its requirements. This may make new functionalities, e.g., data storage and data processing to be designed and easily added to support new use cases.

-	Enhancing interface between AF and 5GC to allow better handling of UPF for header enrichment, as well as direct subscription, detection and reporting/notifications..

This study is to further investigate the issues described above for completing UPF capabilities for exposure and SBA.

4	Objective

The study item will consider the following aspects to support better integration of UPF into the 5GC SBA:

WT#1: Study the enhancement on UPF event exposure service to satisfy the requirements on charging, traffic management, etc. The dependency on other study items needs to be avoided. This will study new requirements on first.

WT#2: Study modular architecture of user plane, including new event exposure services which are supported by existing UPF functionalities e.g. NAT, application detection

WT#3: Study enhancing the interface between AF and 5GC to allow better handling of UPF for header enrichment, as well as direct subscription from AF, detection and reporting/notifications. Analysis of existing and/or new exposure and provisioning functionality/interface(s) is expected.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 0.75 | 0.75 | No | WT#1 is self-contained |
| WT#2 | 0.75 | 0.75 | No | WT#2 is self-contained |
| WT#3 | 1 | 1 | No | WT#3 is self-contained. |

Total TU estimates for the study phase: 2.5

Total TU estimates for the normative phase: 2.5

Total TU estimates: 5

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on UPF enhancement for Exposure And SBA Phase 2 | TSG SA#X (TBD) | TSG SA#Y (TBD) |  |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

TBC

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects, if any, will be addressed by SA3.

Charging aspects, if any, will be addressed by SA5.

9	Supporting Individual Members

Supporting IM name

AT&T

China Mobile

Deutsche Telekom

DISH Network

SK Telecom

Samsung?

ETRI

Rakuten Mobile

Vodafone

ZTE