3GPP TSG-WG SA2 Meeting #158	S2-2309450

# Goteborg, August 21 – 25, 2023				                                                     was S2-2308577

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4292 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for confidence and/or accuracy level associated withUpdate for Application-Specific Expected UE Behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 23.502, it is agreed that each parameter within the Expected UE Behaviour shall have an associated validity time and may have a confidence and/or accuracy level associated with it. These parameters should be applicable to the application-specific expected UE behaviour as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that for Application-Specific Expected UE behavior, the validity time, confidence and/or accuracy level should also be applied and be associated with it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The validity time, confidence and/or accuracy level may not apply to the Application-Specific Expected UE behavior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3f |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 4.15.6.3f	Application-Specific Expected UE Behaviour parameters

These Application-Specific Expected UE Behaviour parameters characterise the foreseen behaviour of a UE for a specific application. The application traffic is identified by Application ID or Packet filters. When AF provisions the Application-Specific Expected UE Behavior parameters, the AF shall provide Application Traffic Information which describes the service flows of the application traffic. It is either an Application ID or Packet Filters. Each parameter within the Application-Specific Expected UE Behaviour shall have an associated validity time and may have a confidence and/or accuracy level associated with it as defined in the clause 4.15.6.2. The confidence level indicates a probability assertion for the associated Application-Specific Expected UE Behaviour parameter and the accuracy level indicates the performance of the estimator (e.g. AI/ML model) used for the prediction.

Table 4.15.6.3f-1: Description of Application-Specific Expected UE Behaviour parameters

| Application-Specific Expected UE Behaviour Parameters | Description |
| --- | --- |
| Expected PDU session Inactivity Time | Identifies the expected PDU Session Inactivity time during which the UE will not have traffic related to the application. |

**** END OF FIRST CHANGE***