3GPP TSG-WG SA2 Meeting #158	S2-2309736

Goteborg, Sweden, August 21 – 25, 2023

Title:	Reply LS on Issues on SMF event exposure and muting enhancements

Release:	Rel-18

Work Item:	eNA_Ph3, AIMLsys

Source:	SA2

To:	CT3

Cc:

Contact person:	Xiaoyan Duan

duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2309586

- Overall description

SA2 thanks CT3 for their LS C3-232578 on issues on SMF event exposure and muting enhancements.

SA2 would like to provide the following answers to the questions from CT3:

Question 1: Which event ID of SMF will be used to collect the 5QI?

Answer to Question 1:

The NWDAF uses the event QFI allocation to collect the 5QI from the SMF as defined in Table 6.9.2-2 and Table 6.18.2-2 of TS 23.288 via Nsmf_EventExposure service. The event QFI allocation is extended to provide 5QI, see the attached agreed S2-2309586.

Question 2: Do the Muting Exception Instructions (and the actions specified in clause 6.2.7 of TS 23.288) apply to such APIs (i.e. APIs that are currently not specified to be used by the NWDAF for data collection) or not?

Answer to Question 2:

The Muting Exception Instructions is defined and used by the NWDAF or DCCF for data collection as specified in clause 6.2.7 of TS 23.288. So far, there is no requirement to apply the Muting Exception Instructions in other scenarios, e.g. event exposure for purposes other than data collection by the NWDAF/DCCF. That is, the Muting Exception Instructions and the actions specified in clause 6.2.7 of TS 23.288 do not apply to APIs that are currently not specified to be used by the NWDAF/DCCF for data collection. See the attached agreed S2-2309586 with clarifications in TS 23.502.

- Actions

To CT3:

ACTION: 	SA2 kindly asks CT3 to take the above answers into account and complete the CT3 specifications accordingly.

3	Dates of next TSG SA WG 2 meetings

3GPP TSG SA2#159	  9th – 13th October 2023	Xiamen, CN

3GPP TSG SA2#160	13th – 17th November 2023	Chicago, US