3GPP TSG-WG SA2 Meeting #158	S2-2309832

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023	(revision of S2-2309489, S2-2309194)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0426 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup about the usage of GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in S2-2307554 in SA2#157, only the Application Layer ID will be used in UE discovery and the GMLC will be used for retrieving the mapping between Application Layer ID and GPSI from NEF. The AF will provision the mapping relationship to UDR to be queried by NEF. The EN in clause 6.20.1 as ” Editor's note: Whether GPSI of other UE can be obtained will be further aligned with SA WG3.” can be removed since GPSI will not be used here. For the SL-MT-LR procedure in clause 6.20.3, if only GPSI is provided, the GMLC should query the NEF to get the mapping Application Layer ID for the upcoming step 12 for UE discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the EN according to the consensus achieved in SA2 #157. - Update clause 6.20.3 to support the GPSI only scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear about how to use the GPSI and Application Layer ID in SL-MO-LR and SL-MT-LR procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20.1, 6.20.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.586 CR 0036 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.20.1	Procedures of SL-MO-LR involving LMF

Figure 6.20.1-1 illustrates a procedure to enable a UE to obtain Sidelink positioning/ranging location results using one or more other UEs with the assistance of an LMF in a serving PLMN for UE1.

The Ranging/SL Positioning location results may include absolute locations, relative locations or distances and directions, depending on the service request.

If the Target UE decides to initiate SL-MO-LR procedure, it includes one or multiple SL reference UE(s) / Located UE (s) in the service request. See TS 23.586 [40] for more information on how this generic procedure can be used.

Figure 6.20.1-1: SL-MO-LR Procedure

Precondition:	UE1 is in coverage and registered with a serving PLMN. UEs 2 to n may or may not be in coverage and, if in coverage, may or may not be registered with the same serving PLMN as UE1.

1.	The procedures and signalling specified in clause 6.2 of TS 23.586 [40] may be used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to UEs 1 to n, when in coverage.

NOTE 1:	If indication of UE-only operation is received, procedures of Ranging/Sidelink Positioning control as defined in clause 6.8 of TS 23.586 [40] is performed.

2.	Based on a trigger of service request (e.g. received from the application layer), which includes UE1/.../UEn, UE discovery is performed for Ranging/SL positioning as specified in clause 6.4 of TS 23.586 [40]:

-	If UE1 is the target UE, UE1 discovers UEs 2 to n

-	If UE1 is the Located UE, the target UE (i.e. one of the UEs 2 to n) discovers UE1 (and other Located UEs in the set of UEs 2 to n).

3.	Secure groupcast and/or unicast links are established between UEs 1 to n as defined in clause 5.3 of TS 23.586 [40] to enable UE1 to exchange Ranging and Sidelink Positioning Protocol (RSPP) messages over PC5-U reference point with each of UEs 2 to n and possibly enabling UEs 2 to n to exchange RSPP over PC5-U between each other.

Editor's note:	Security aspects for RSPP signalling need to be defined or agreed by SA WG3.

4.	UE1 and UEs 2 to n may communicate over PC5 for authorization of Ranging/SL positioning and receiving QoS parameters if needed. Each of UEs verifies that Ranging/SL positioning is permitted, including whether Ranging/SL positioning results may be transferred to an LCS Client or AF if this is used, according to any service authorization and policy/parameter provisioning received at step 1. QoS requirements for the Ranging/SL positioning may be also provided based on QoS requirements in the service request.

5.	UE1 may obtain the Sidelink positioning capabilities of UEs 2 to n using the groupcast and/or unicast links established in step 3.

Step 4 and 5 may be performed to transfer the information of UEs which are not served by the LMF.

NOTE 2:	UE2/.../UEn is not assumed to be served by the same LMF serving UE1.

Editor's note:	It needs to be verified that RAN2 will provide support for steps 4 and 5.

6.	Based on the Sidelink positioning capabilities of UE1/.../UEn, the target UE determines SL-MO-LR is to be performed. If UE1 is the Located UE (i.e. when the target UE is one of UE2/.../UEn and does not have NAS connection), the target UE initiates SL-MO-LR service request to UE1.

7.	If UE1 is in CM-IDLE state, UE1 instigates a UE triggered Service Request in order to establish a signalling connection with the serving AMF of UE1.

8.	UE1 sends a supplementary services SL-MO-LR request to the serving AMF in an UL NAS TRANSPORT message. The SL-MO-LR request indicates the other UEs 2 to n (using application layer ID), indicates any assistance data needed, indicates whether location calculation assistance is needed, and indicates whether location results should be transferred to an LCS client or AF. The message shall include the identity of the LCS client or the AF and may include the address of the GMLC through which the LCS client or AF (via NEF) should be accessed. In addition, a Service Type indicates which MO-LR service of the LCS Client is requested by the UE may be included. For location calculation assistance from the LMF, the preferred type of Sidelink positioning/ranging location results (e.g. absolute locations, relative locations or distances and directions between pairs of UEs) and the required QoS are included. If UE1 is Located UE and one of UE2/.../UEn is the target UE that does not have NAS connection, the supplementary services SL-MO-LR request includes an indication that one of UE2/.../UEn is the target UE instead of UE1.

Editor's note:	It is FFS whether indication about the need of assistance data from UE, which will be aligned with RAN WGs.

9.	The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Sidelink positioning/ranging) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information from the SL-MO-LR Request. The service operation includes a LCS Correlation identifier.

10.	The LMF sends a request to UE1 for the capabilities of UEs 1 to n.

NOTE 3:	UE2/.../UEn is not assumed to be served by the same LMF serving UE1.

Editor's note:	It is FFS the update when the LMF also serves UE2/.../UEn with alignment with RAN.

11.	UE 1 returns its capabilities to the LMF. UE1 may additionally return the capabilities of the UEs obtained at step 5 if requested by the LMF at step 10.

12.	UE1 may send a request for specific assistance data to the LMF.

13.	LMF sends the requested assistance data to UE1, and UE1 forwards the assistance data received from LMF to UE2/.../UEn. The assistance data may assist UEs 1 to n to obtain Sidelink location measurements at step 15 and/or may assist UE1 to calculate Sidelink positioning/ranging location results at step 16.

NOTE 4:	Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8.

Editor's note:	Whether step 10-11 are needed will be aligned with RAN WGs.

14.	If the SL-MO-LR request at step 8 indicated location calculation assistance is needed and/or indicated transfer of Sidelink positioning/ranging location results to an LCS Client or AF, the LMF sends a request for location information to UE1 and may also send a request for location information to UE2/.../UEn if it is served by the LMF. If LMF determines to apply UE based SL Positioning, LMF includes in the request the indication of UE based SL Positioning. LMF may also provide the list of candidate Located UE(s), if absolute location is requested at step 8. If scheduled location time is received at step 14. LMF may include a scheduled location time.

Editor's note:	It needs to be aligned with RAN WG2 if the list of candidate Located UE(s) is provided in step 13 or step 14.

15.	UE1 instigates a Sidelink positioning/ranging procedure among UEs 1 to n in which UEs 1 to n obtain Sidelink location measurements and UEs 2 to n transfer their Sidelink location measurements to UE 1 and/or to the LMF (depending on the assistance requested). If scheduled location time is received at step 14, Sidelink positioning/ranging is performed at the scheduled location time.

16. If Target UE's absolute location information is required at step 8 and if absolute location of Located UE(s) is not available, the Target UE sends a request to the Located UE(s) to trigger 5GC-MO-LR procedure to let the Located UE(s) acquire their own absolute location. The QoS requirement received at step 8 is included in the request, which is used to derive the QoS for Located UE(s) positioning.

17.	If LMF determines to use UE based calculation, at least one of UE1/.../UEn calculates Sidelink positioning/ranging location results based on the Sidelink location measurements obtained at step 15 and possibly using assistance data received at step 13. The Sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n.

18.	If UE1 received a request for location information at step 14, UE1 sends a response to the LMF and includes the Sidelink location measurements obtained at step 15, the Sidelink positioning/ranging location results obtained at step 17 if step 17 was performed, or Located UE's absolute location obtained at step 16.

19.	If Target UE's absolute location information is required at step 8 and if absolute location of Located UE(s) is not received at step 18, LMF can either retrieved the location of the Located UE(s) locally or triggers 5GC-MT-LR procedure to the GMLC to acquire the absolute location of the Located UE(s) using Application Layer ID of the Located UE(s). LMF includes the QoS requirement received at step 8 in the request, which is used to derive the QoS for Located UE(s) positioning. If scheduled location time is used, LMF includes the scheduled location time in the request to GMLC.

20.	The LMF calculates Sidelink positioning/ranging location results for UEs 1 to n from the Sidelink location measurements received at step 18 and absolute location of Located UE(s) at step 19. The Sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n, depending on the location request received in step 8.

21.	The LMF returns an Nlmf_Location_DetermineLocation service operation response to the AMF and includes the Sidelink positioning/ranging location results received at step 18 or calculated at step 20.

22.	If Sidelink positioning/ranging location results were received at step 21, the AMF performs steps 7-12 of clause 6.2 to send the Sidelink positioning/ranging location results to the GMLC and to an AF or LCS Client if this was requested at step 8. The Sidelink positioning/ranging location results include the identities for the respective UEs 1 to n received at step 8.

NOTE 5:	Sending location results and global identities for UEs 1 to n to an AF or LCS Client may require privacy verification from UEs 1 to n and/or from the HPLMNs of UEs 1 to n.

23.	The LMF returns a supplementary services SL-MO-LR response to UE1 in a DL NAS TRANSPORT message and includes any Sidelink positioning/ranging location results calculated at step 20 if step 20 was performed. If UE1 is Located UE, and the target UE is one of the UEs 2 to n and does not have NAS connection, then UE1 may transfer the Sidelink positioning/ranging location results to the target UE.

# * * * * Second change * * * *

6.20.3	Procedures of SL-MT-LR involving LMF

The SL-MT-LR procedure is used to estimate the relative locations or distances and/or directions between the UEs.

Figure 6.20.3-1 illustrates a procedure to enable an LCS Client or AF to obtain Ranging/Sidelink Positioning location results for a group of n UEs (n≥2), i.e. UE1, UE2, ..., UEn. In the procedure, the GMLC determines a UE among the n UEs to be designated UE1 (i.e. Target UE in TS 23.586 [40]) and one or more other UEs designated UE2, UE3, ..., UEn (n≥2) (i.e. Reference/Located UEs in TS 23.586 [40]). The Ranging/Sidelink Positioning location results may include absolute locations, relative locations or ranges and directions related to the UEs, based on the service request.

Procedure for periodic and triggered SL-MT-LR is defined in clause 6.20.4.

Figure 6.20.3-1: SL-MT-LR Procedure

Precondition:	At least one of the n UEs is in coverage and registered with a serving PLMN.

1.	The LCS Client or the AF (via NEF) sends an LCS service request to the (H)GMLC for Ranging/Sidelink Positioning location results for the n UEs which may each be identified by an Application Layer ID, a GPSI or a SUPI. The request may include the required QoS, the required location results (e.g. absolute locations, relative locations or distances and/or directions related to the UEs), the SL reference UE(s) in case of relative locations, distance, or direction. The (H)GMLC or NEF authorizes the LCS Client or the AF for the usage of the LCS service. If the authorization fails, the remaining steps are skipped and the (H)GMLC or NEF responds to the LCS Client or the AF with the failure of the service authorization.

In addition, an Application Layer ID shall be used for each of the n UEs to enable discovery of the UEs at step 12. If only GPSI is provided to the (H)GMLC, it can query the NEF for the mapping Application Layer ID as specified in 4.3.9 of TS 23.586 [40].

Editor's note:	Whether Application Layer ID will be included in LPP operation is FFS and needs the coordination with RAN WGs.

2.	The (H)GMLC invokes a Nudm_SDM_Get service operation towards the UDM of each of the n UEs to get the privacy settings of the UE identified by its GPSI or SUPI. The UDM returns the UE Privacy setting of the UE. The (H)GMLC checks the UE LCS privacy profile.

3.	The (H)GMLC invokes a Nudm_UECM_Get service operation towards the UDM of each of the n UEs (for which GPSI or SUPI is available), one at a time, using the GPSI or SUPI of each UE. The (H)GMLC selects the UE (e.g. which is treated as UE1 in following steps) that initiates the Ranging/SL Positioning and selects the corresponding serving AMF.

NOTE:	The UDM is aware of the serving AMF address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19]. The UDM is aware of a serving (V)GMLC address at UE registration on an AMF as defined in clause 4.2.2.2.2 of TS 23.502 [19].

4.	For a non-roaming case, this step is skipped. In the case of roaming, the (H)GMLC may receive an address of a (V)GMLC (together with the network address of the current serving AMF) from the UDM in step 3, otherwise, the (H)GMLC may use the NRF service in the (H)PLMN to select an available (V)GMLC in the (V)PLMN, based on the (V)PLMN identification contained in the AMF address received in step 3. The (H)GMLC then sends the location request to the (V)GMLC by invoking the Ngmlc_Location_ProvideLocation service operation towards the (V)GMLC. In the cases when the (H)GMLC did not receive the address of the (V)GMLC, or when the (V)GMLC address is the same as the (H)GMLC address, or when both PLMN operators agree, the (H)GMLC sends the location service request message to the serving AMF. In this case, step 4 is skipped. The (H)-GMLC also provides the LCS client type of AF, if received in step 1, or LCS client type of LCS client and other attributes to be sent to AMF in step 5.

5.	In the case of roaming, the (V)GMLC first authorizes that the location request is allowed from this (H)GMLC, PLMN or from this country. If not, an error response is returned. The (H)GMLC or (V)GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving UE1 to request Sidelink positioning/ranging location results of the n UEs. The service operation includes the SUPI of UE1, Application layer IDs of the UEs, the client type and may include the required LCS QoS, the required location results (e.g. relative locations or ranges and directions related to the UEs) and other attributes as received or determined in step 1.

6.	If UE1 is in CM-IDLE state, the AMF initiates a network triggered Service Request procedure to establish a signalling connection with UE1.

If signalling connection establishment fails, steps 7-17 are skipped.

7-8.	If the indicator of privacy check indicates an action is needed, then same operation as that of step 7-8 of clause 6.1.2 is carried out.

9.	The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink Positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information received at step 5 e.g. required location results (e.g. relative locations or ranges and directions between pairs of UEs), SL reference UE(s) in case of relative locations, Application layer IDs of the UEs if received in step 5. The service operation includes a LCS Correlation identifier.

10.	The LMF sends an SL-MT-LR request to the serving AMF as a supplementary services message, using the Namf_Communication_N1N2MessageTransfer service operation, and the session ID parameter is set to the LCS Correlation identifier.

The SL-MT-LR request may include the application layer IDs of the other UEs 2 to n, the types of required location results (e.g. relative locations or distances and/or directions) and SL reference UE(s) in case of relative locations.

Editor's note:	Whether supplementary services message or lpp will be sent from LMF to UE is FFS and needs the coordination with RAN WGs.

11.	The serving AMF forwards the SL-MT-LR request and a Routing identifier equal to the LCS Correlation identifier to UE1 using a DL NAS TRANSPORT message.

12.	UE1 attempts to discover the other UE 2 to n using their Application Layer IDs if not already discovered using procedure defined in clause 6.4 of TS 23.586 [40].

13.	UE1 obtains the sidelink positioning capabilities of the discovered UEs via the SLPP if not already obtained.

14.	UE1 returns a supplementary services SL-MT-LR response to the serving AMF in an UL NAS TRANSPORT message and includes the Routing identifier received in step 11.

The SL-MT-LR response indicates which of UEs 2 to n have been discovered and the sidelink positioning capabilities of the discovered UEs.

15.	The serving AMF forwards the SL-MT-LR response to the LMF indicated by the Routing identifier received at step 14 and includes a LCS Correlation identifier equal to the Routing identifier.

16.	Ranging/Sidelink Positioning of UE1 and the other discovered UEs occurs as for an SL-MO-LR as described for steps 10-19 of Figure 6.20.1-1 with the difference that Ranging/Sidelink Positioning location measurement data or results are always returned to the LMF and the LMF indicates to UE1 at step 13 or step 14 of Figure 6.20.1-1 whether the Ranging/Sidelink Positioning location results will be calculated by the LMF (at step 19) or by UE1 (at step 17). For some undiscovered UEs among the other UEs 2 to n, the LMF interacts with GMLC to initiate the 5GC-MT-LR procedure for UE2 to n to get their absolute locations, and calculates the relative locations or distances and/or directions related to the UEs.

17-20.	The LMF returns the Sidelink positioning/ranging location results to the LCS Client or AF as in steps 13-15 and step 24 of clause 6.1.2. The results also include failure information of the UE(s) that was not discovered.

# * * * * End of changes * * * *