3GPP TSG-WG SA2 Meeting #158	S2-2310023

# Gothenburg Aug 21 – Aug 25, 2023

# Title:	(DRAFT) Reply LS on periodic attempts for re-selection to a higher priority SNPN when access for localized services in SNPN is enabled

# Response to:	(S2-2308277/C1-232888) LS on periodic attempts for re-selection to a higher priority SNPN when access for localized services in SNPN is enabled

# Release:	Rel-18

# Work Item:	eNPN_Ph2

Source:	SA2

To:	CT1

Cc:	SA1

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter dot hedman at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

SA1 replied to CT1’s question in S1-231752, that there is no service requirement on periodic attempts to reselect higher priority SNPN.

In relation to the CT1 LS, SA2 has discussed the scenario described below:

-	In Figure-1 below, when the UE is in location 1, the UE remains in subscribed SNPN even after the end user has enabled the access to localized service and validity condition changes from not met to met (as the hosting network is not in reach for the UE).

-	When the UE later moves to hosting network area, the UE should be able to automatically select the hosting network SNPN.

Figure 1 Periodic reselection

SA2 address the above scenario by the following in TS 23.501 clause “5.30.2.4.2 Automatic network selection”:

"If the UE supports accessing an SNPN providing access for Localized Services and the end user enables to access Localized Services, the UE shall periodically attempt reselection and registration on a higher priority SNPN 1) based on the order of the above sub-bullets… “

SA2 kindly asks CT1 to address the scenario described above in their specifications

2. Actions:

To CT1:

ACTION	SA2 would like CT1 to take the above into consideration and address the above scenario.

3. Date of Next SA2 Meetings:

SA2#159	9th – 13th October 2023 		Xiamen, China

SA2#160	13th – 17th November 2023 		Chicago, USA