3GPP TSG-SA2 Meeting #159	S2-2310247

# Xiamen, China, October 09 - October 13

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0922 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on movement behaviour analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | First, step 1 of Figure 6.21.4-1 has an incorrect name (i.e. Traffic flow) for Analytics ID and step number (i.e. 1b). Second, a few terms (i.e. users and AOI) may lead to confusion. Last, editorial updates for clarification are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.21.1 is updated to clarify that the term of “AOI” in this analytics only refers to fine granularity location information, in contrast to other analytics where AOI is defined by TA and/or Cell levels. Clause 6.21.3 is updated to replace the term “user” by “UE”. Clause 6.21.4 is updated to correct an incorrect name (i.e. Traffic flow->movement behavior) of for Analytics ID and step number (i.e. 1b->1) in step 1 of Figure 6.21.4-1. All the clauses are partly updated for editorial corrections or clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21.1, 6.21.3, 6.21.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.21	Movement Behaviour Analytics

6.21.1	General

The movement behaviour analytics provides analytics information (statistics or predictions) regarding the location, direction and velocity of UEs during an analytics target period in a target area. The analytics of movement behaviour requires to collect a map of UE movements in a certain region in a certain period of time. In addition, the consumers can obtain the congestion situation in the target area through statistics.

The NWDAF provides movement behaviour analytics to a NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [39] as this analytics is basically derived based on fine granularity location information;

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.21.3);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

NEXT CHANGE

6.21.3	Output analytics

The output for analytics of movement behaviour provided by NWDAF is defined in Table 6.21.3-1 and Table 6.21.3-2.

Table 6.21.3-1: movement behaviour statistics

| Information | Description |
| --- | --- |
| Applicable Area | A geographical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time period for UE location information collection from the beginning to the end |
| > Total number of UEs (NOTE) | Total number of UEs in the area of interest |
| > Ratio of moving UEs (NOTE) | Ratio of moving UEs in the area of interest |
| > Average speed (NOTE) | Average speed of all UEs in the area of interest |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to filter the UEs |
| >> Number of UEs | The number of UEs whose speed is faster than the speed threshold |
| >> Ratio | Percentage of UEs whose speed is faster than the speed threshold |
| > Direction (1…max) (NOTE) | Heading directions of the UEs in the target area, such as north, south, east, west etc. |
| >> Number of UEs | The number of UEs in the specific direction |
| >> Average speed | Average speed of UEs in the specific direction |
| >> Ratio | Percentage of UEs in the specific direction |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.21.3-2: movement behaviour predictions

| Information | Description |
| --- | --- |
| Applicable Area | A geographical area as defined in TS 23.273 [39] that the analytics applies to |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time period for UE location information collection from from the beginning to the end |
| > Total number of UEs (NOTE) | Total number of UEs in the area of interest |
| > Ratio of moving UEs (NOTE) | Ratio of moving UEs in the area of interest |
| > Average speed (NOTE) | Average speed of all UEs in the area of interest |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to filter the UEs |
| >> Number of UEs | The number of UEs whose speed is faster than the speed threshold |
| >> Ratio | Percentage of UEs whose speed is faster than the speed threshold |
| > Direction (1..max) (NOTE) | Heading directions of the UEs in the target area, such as north, south, east, west etc. |
| >> Number of UEs | The number of UEs in the specific direction |
| >> Average speed | Average speed of UEs in the specific direction |
| >> Ratio | Percentage of UEs in the specific direction |
| Confidence | Confidence of the prediction |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

NEXT CHANGE

6.21.4	Procedures

Figure 6.21.4-1 depicts the procedure for movement behaviour analytics provided by NWDAF.

Figure 6.21.4-1: "Movement Behaviour" analytics provided by NWDAF

1.	The NWDAF receives an analytics request from the NWDAF service consumer (as defined in clause 6.1.1.1) or from an AF via NEF (as defined in clause 6.1.1.2) to request movement behaviour analytics within a target area for any UE.

The Analytics ID is set to "Movement Behaviour". The target for analytics reporting is set to be any UE. Analytic filters may be provided as shown in clause 6.21.1. The Area of Interest is the target area that may be defined as a geographical area or as a geopolitical name of an area, in such case, the NEF will translate it as the identities of one or more radio cells or tracking areas.

The NWDAF service consumer NF can request statistics or predictions or both for a given Analytics target period.

2.	The NWDAF collects input data from AMF as defined in Table 6.21.2-1 via the AMF event exposure service.

3.	The NWDAF collects the UE location information as defined in Table 6.21.2-1 via Ngmlc_Location services.

Based on the analytics target period, the NWDAF may trigger the immediate 5GC-MT-LR procedure as defined in clause 6.1.2 or the deferred 5GC-MT-LR procedure in clause 6.3.1 of TS 23.273 [39] to obtain UE's location information.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics as defined in Table 6.21.3-1 and Table 6.21.3-2 using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription _Notify, depending on the service used in step 1.

END CHANGE