SA WG2 Meeting #159	S2-2310306

# Xiamen, PRC, October 9-13th, 2023        	  	  			       		            (revision of S2-231)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4467 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Achieved sampling ratio in Nupf_EventExposure_Notify to align to 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 23.288 indicates that UPF is able to indicate the Sampling ratio achieved by UPF Table 6.8.2-2: Data Collected from the UPF or from the AF related to User Data Congestion Analytics Information; Source; Description. Application ID; UPF or AF; Application identifier as defined in clause 5.8.2 of TS 23.501 [2] (see NOTE 1).. IP Packet Filter Set; UPF or AF; IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 [2] (see NOTE 1).. Measurement period; UPF or AF; Measurement period.. Throughput UL/DL; UPF or AF; Average Throughput UL/DL over the measurement period.. Throughput UL/DL (peak); UPF or AF; Peak Throughput UL/DL over the measurement period.. Timestamp; UPF or AF; Time when measurements are taken.. Achieved sampling ratio; UPF; Sampling ratio achieved by UPF (see NOTE 2).. NOTE 1: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 2: UPF may apply data sampling to reduce the load on the UPF. This parameter is provided when no sampling ratio is configured at the UPF or the UPF could not fulfil the configured sampling ratio. NOTE 3: Multiple outputs are provided by the UPF when multiple Service Data Flows are running at the UPF for the same UE and measurement period.. |
| Summary of change: |  | Adding Achieved sampling ratio in Nupf_EventExposure_Notify service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 23.502 service operation is not aligned with 23.288 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.26.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed implicitly.

Input Required: Event ID, UE address (i.e. IP address or MAC address). Notification Correlation Information.

Input, Optional: UE ID, Event specific parameter as described in clause 5.2.26.2.1, time stamps for the measures, Application Id and Packet Filter Set, Achieved sampling ratio.

Output Required: Result Indication.

Output, Optional: None.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES