3GPP SA WG2 Meeting #159	S2-2310870

Xiamen, PRC, October 9-13th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Scope for TR 23.700-70

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes scope for TR 23.700-70, XRM_Ph2

# Discussion

SP-231198 includes the SA approved SID for FS_XRM Ph2.

Scope is proposed based on approved objectives.

# Proposal

It is proposed to include scope as follows in the XRM Ph2 TR.

*** Start of changes ***

# 1	Scope

The Technical Report studies key issues, solutions and conclusions for support of advanced media services, e.g. High Data Rate Low Latency (HDRLL) services, AR/VR/XR services. The objectives include:

Enhancement for PDU Set based QoS handling.

- Study whether and how to enhance PDU Set related (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC) and PDU Set information (including Control Plane and/or User plane information provided by the AF/AS) and the corresponding PDU Set QoS handling enhancement.

NOTE 1: This will require close coordination between SA4 and SA2.

- Support QoS control and PDU Set identification for XR stream with e2e encryption (e.g. fully encrypted header, partially encrypted header). This is applicable for PDUs received at N6 for DL.

- Enhancements to support PDU Set based QoS handling in uplink direction.

- Study whether and how to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

QoS handling enhancement for XRM services.

- Study whether and what enhancements are needed for traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

- Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

NOTE 2: This will require close coordination between SA4 and SA2.

- Study whether and how to identify traffic flows and study whether and how QoS handling enhancement may be needed for the UE with the tethered devices for the uplink traffic (for example, traffic from tethered devices mapped to different QoS Flows enabling QoS differentiation such as PDU Set based QoS flows for XR traffic for device 1 vs PDU based QoS flows for eMBB traffic for device 2).

NOTE 3: The interface between 3GPP UE and tethered devices behind the UE is outside of scope.

Further enhancement to support XR based on non-3GPP access.

- Study how to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.

-	Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF).

-	Support L4S in wireline access (e.g. W-AGF).

- Study how PDU Set QoS Control mechanisms can be extended to non-3GPP access networks.

-	Support PDU Set QoS in untrusted/trusted access (e.g. N3IWF, TNGF).

-	Support PDU Set QoS in wireline access (e.g. W-AGF).

NOTE 4: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

Network exposure: Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

NOTE 5:  Alignment and coordination with RAN work will be needed for the study.