3GPP SA WG2 Meeting #159	S2-2310874

Xiamen, PRC, October 9-13th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	KI description for WT#1.4

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes KI description for WT#1.4

# Discussion

SP-231198 defined FS_XRM Ph2 WT#1.4 for Rel. 19 as:

WT#1 Enhancement for PDU Set based QoS handling.

WT#1.4 Study whether and how to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

A Rel. 19 KI needs to be defined for these WT. The following is proposed.

# Proposal

It is proposed to define the following KI for Rel. for XRM_Ph2 WT#1 for incorporation in the XRM Ph2 TR23.700-70.

*** Start of changes ***

## X.1	Key Issue #X: Leveraging PDU Set Information for DSCP marking over N3/N9

## X.1.1	Description

This key issue studies whether and how to leverage PDU Set Information for Differentiated Services Code Point (DSCP) marking over N3/N9 in the transport network (i.e., to enable differentiated handling of PDU Sets within QoS Flow).

In Rel-18, DSCP marking is at the QoS flow level meaning every PDU in a QoS flow is marked with the same DSCP regardless of PDU Set information. The marking is performed based on instructions from the SMF.

The objective of this key Issue is to investigate whether and how PDU delivery in the transport network based on DSCP can be affected by PDU Set information for the various PDU Sets within a QoS flow.

This key issue will study:

-  	Whether and how PDU Set information based DSCP marking can be done for the 3GPP transport network on N3/N9.

-  	Whether and how the rules provided by SMF to instruct DSCP marking should be extended to allow the control of PDU Set information dependent DSCP marking.