3GPP TSG-SA WG2 Meeting #159	S2-2311432

# Xiamen, China, October 09 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0115 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to authorization for Direct C2 communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 5.4.3.2 (Direct C2 communication authorization during UUAA-MM), it only describes the successful direct C2 authorization and is missing the case where the direct C2 authorization fails. Also it does not describe to send the C2 authorization result by the USS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the authorization request for Direct C2 communication was sent by the UE, the USS sends the C2 authorization result and optionally the C2 authorization payload containing direct C2 pairing information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If direct C2 authorization fails, the result is not provided to the UE and then the UE may retry via UUAA-SM unnecessarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.4.3.2	Authorization of Direct C2 Communication service via UUAA-MM

In 5GS, the UUAA-MM procedure may be used to authorize direct C2 communication described in clause 5.2.2 with the following enhancements:

-	In step 1 of clause 5.2.2.1: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the UAV may include an authorization request for Direct C2 Communication by providing a C2 Aviation Payload with an indication that the authorization is for Direct C2 Communication. In addition, the UAV includes the direct C2 pairing information (if available) in the C2 Aviation Payload.

-	In steps 2 and 3 of clause 5.2.2.2: if the UAV included the C2 Aviation Payload in step 1 of clause 5.2.2.1, the C2 Aviation Payload is also included

-	In steps 5 and 6 of clause 5.2.2.2: If the authorization request for Direct C2 Communication was included in step 1 of clause 5.2.2.1, the USS sends the C2 Authorization result and, in case of success, may include a direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload, which are further forwarded to the UAV/UE.

If the UAV attempts Direct C2 Communication authorization via UUAA-MM and receives no response, the UAV shall perform Direct C2 Communication via UUAA-SM.

End of CHANGE