3GPP TSG-WG SA2 Meeting #159	S2-2311502

# Xiamen, China, October 9 – 13, 2023		(revision of S2-2310481)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2116 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behind 5G-RG and FN-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shaghai Bell, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture of support of UE behind the FN-RG/5G-RG to access to 5GC via untrusted N3GPP access is not technically correct since the FN-RG cannot support N1 interface. Therefore, the figure should be corrected. Additionally, some descriptions are technically incorrect. Therefore, further clarification is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the figure 4.10.2 with two figures corresponding to architectures of supporting UE behind FN-RG and 5G-RG to access to 5GC via untrusted N3GPP access. Moreover, clarification is added accordingly. Remove NOTE 7 which is the duplication of NOTE 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text and the figures are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.10	UE behind 5G-RG and FN-RG

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an N3IWF or TNGF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

Figure 4.10-2a shows the non-roaming architecture for a UE, behind a FN-RG, accessing the 5GC via N3IWF.

Figure 4.10-2b shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via N3IWF.

Annex A shows the non-roaming architecture for a UE, behind a FN-RG/5G-RG, accessing the 5GC via N3IWF where the combination of FN-RG/5G-RG, W-5GAN and UPF serving the 5G-RG is acting as an untrusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG is not 5G capable and therefore it does not support Ta reference point.

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via trusted non-3GPP access with 5G-RG acting as TNAP, NG-RAN or via both accesses.

Figure 4.10-2a: Architecture for UE behind FN-RG using untrusted N3GPP access

Figure 4.10-2b: Architecture for UE behind 5G-RG using untrusted N3GPP access

The FN-RG can only be connected to 5GC via W-5GAN. The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via untrusted non-3GPP access with FN-RG/5G-RG acting as WLAN access point, NG-RAN or via both accesses.

The TNGF and Ta reference point are defined in TS 23.501 [2]. In addition to the requirements described in TS 23.501 [2], the Ta reference point should be able to carry the TNAP ID to the TNGF.

NOTE 2:	The reference architecture in figure 4.10-1/4.10-2a/4.10-2b only shows the architecture and the network functions directly connected to W-5GAN or TNGF/N3IWF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1/4.10-2b apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

NOTE 5:	For trusted non-3GPP access, UE connects to the overlay 5G network using the trusted non-3GPP access approach. In addition to being connected to the underlay 5G network, the 5G-RG also acts as TNAP with respect to the TNGF in the overlay network i.e. it has an established Ta reference point with the TNGF.

NOTE 6:	Support for QoS differentiation can be achieved in a similar way as it is handled when a UE connects to a PLMN via SNPN (as defined in clauses 5.30.2.7 and D.7 of TS 23.501 [2]). Also differentiated charging, both in the RG's PLMN and in the UE's PLMN, can be achieved based on existing mechanisms. This is further described in Annex B.

Support of NSWO for 3GPP UE behind an RG is specified in clause 4.10d.

A 5G-RG acting as a TNAP shall provide its TNAP ID. to the TNGF and the TNGF provides this TNAP ID as part of ULI (User Location Information) sent to the 5GC; this information is propagated to the PCF that may use it to determine PCC rules depending on whether an UE is using a 5G-RG as a host or as a guest.

NOTE 8:	QoS and charging differentiation based on user location (e.g. home or guest users) can be applied when the user is connected via a TNGF reached over a 5G-RG. The PCF may use the TNAP ID, which is available to it as a part of ULI. For example, if the TNAP ID is included in the UE's policy control subscription information the UE is considered a home user. Alternatively, the PCF may use TNAP ID provided by an AF using the Service Specific parameter provisioning as defined in clause 9.8.

# * * * * End of changes * * * *