3GPP TSG SA WG2 Meeting #159	S2-2311803

# Xiamen, China, October 9 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2122 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access restriction for AUN3 devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Charter Communications, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR addresses the following issues in the TS: Issue #1: Connection of AUN3 devices behind 5G RG is currently supported by the specification. However, it is not clear how access and mobility restriction is applied to them, which needs to be clarified in the standard. Furthermore, it is stated in TS. 23.316 that “5GS specifications do not support a device using the same subscription to access 5GS as a UE and as an AUN3 device.” This requires distinguishing between the subscription data of the AUN3 and other devices to ensure that only AUN3 device subscriptions can register using AUN3 device procedures. Issue #2: In case the 5G-RG is deregistered, the W-AGF will release the N2 connection of associated AUN3 devices and provide a cause to AMF indicating that the 5G-RG has been released. In this case the AMF should deregister the AUN3 device. The AMF should include in the deregistration message the reason/cause (5G-RG disconnected) so that the 5G-RG can take the appriate actions (e.g. re-register itself). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Access and mobility policy is added for AUN3 devices behind RG - A parameter is added to the subscription data to enable restriction of access of the AUN3 devices. - It is specified that the deregistration message of AUN3 devices when N2 connection is released includes the cause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The access control for the AUN3 devices will be unspecified and the specification will be imcomplete in this regard. The cause code for deregistration of AUN3 devices when 5G-RG deregisters is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5, 8.1.1.1, 7.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change (Issue #1) * * * *

## 9.5	Non-session management related policy information

### 9.5.1	Access and mobility related policy information

This clause specifies the delta related to Access and Mobility related policy information defined in TS 23.503 [4] clause 6.1.2.1 for 5G-RG and FN-RG.

The access and mobility policy control encompass the management of service area restrictions for a 5G-BRG or a 5G-CRG connecting to 5GC via W-5GAN or simultaneously via NG-RAN and W-5GAN as well as AUN3 devices behind a 5G-RG.

The management of service area restrictions enables the PCF of the serving PLMN or SNPN to modify the service area restrictions based on operator defined policies at any time, either by expanding a list of allowed GLIs or HFC Node IDs or by reducing the list of non-allowed GLIs or HFC Node IDs. Operator defined policies in the PCF may depend on input data such as time of day, or UE context information provided by other NFs, etc.

The AMF reports the subscribed service area restrictions in NG-RAN received from UDM during 5G-RG Registration in NG-RAN procedure when local policies in the AMF indicate that Access and Mobility Control is enable within the PLMN or SNPN. The AMF may request update of the service area restrictions applicable to NG-RAN when the policy control request triggers listed in clause 6.1.2.5 in TS 23.503 [4], are met.

The AMF reports the subscribed service area restrictions in W-5GAN received from UDM during 5G-RG or AUN3 device Registration in W-5GAN procedure when local policies in the AMF indicate that Access and Mobility Control is enable within the PLMN or SNPN. The AMF may request update of the service area restrictions applicable to W-5GAN when the policy control request triggers listed in clause 9.5.3, are met.

The AMF receives the modified service area restrictions from the PCF and then use them as described in clause 4.3.3.3.

No mobility events, such a change of UE location or change of AMF applies when provisioning the service area restrictions for a 5G-BRG or a 5G-CRG or AUN3 device when connected via W-5GAN.

The PCF may provide the service area restrictions applicable to a 5G-RG connected to 5GC via W-5GAN or via NG-RAN or simultaneously connected to 5GC via W-5GAN and NG-RAN to AMF. The PCF may provide the service area restrictions applicable to a FN-CRG to the AMF. The PCF may provide the service area restrictions applicable to an AUN3 device behind 5G-RG connected to 5GC via W-5GAN to the AMF.

The Service Area Restrictions provided to AMF for a 5G-RG connected via NG-RAN is according to the information listed in listed in TS 23.503 [4] clause 6.5.

The Service Area Restrictions provided to AMF for a 5G-RG or AUN3 device connected via W-5GAN is according to the information listed in Table 9.5-1.

For a 5G-RG simultaneously connected to 5GC via W-5GAN and NG-RAN the PCF provides Service Area Restrictions for both W-5GAN and NG-RAN.

The Service Area Restrictions provided to AMF for a FN-CRG connected via W-5GAN is according to the information listed in Table 9.5-1.

Table 9.5-1: Access and mobility related policy control information for 5G-RG and FN-CRG accessing via W-5GAN

| Information name | Description | Category | PCF permitted to modify in a AM context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| Service Area Restrictions for a 5G-BRG or AUN3 device behind 5G-BRG | This part defines the service area restrictions applicable for a 5G-BRG or for an AUN3 device behind 5G-BRG |  |  |  |
| List of allowed GLIs | List of allowed GLIs (NOTE 2). | Conditional (NOTE 1) | Yes | AM context |
| List of non-allowed GLIs | List of non-allowed GLIs . | Conditional (NOTE 1) | Yes | AM context |
| Service Area Restrictions for a 5G-CRG or for a FN-CRG | This part defines the service area restrictions applicable for a 5G-CRG or for a FN-CRG |  |  |  |
| List of allowed HFC Node IDs | List of allowed HFC Node IDs (NOTE 2) | Conditional (NOTE 1) | Yes | AM context |
| List of non-allowed HFC Node IDs | List of non-allowed HFC Node IDs (NOTE 2). | Conditional (NOTE 1) | Yes | AM context |
| Service Area Restrictions for an AUN3 device behind 5G-CRG | This part defines the service area restrictions applicable for an AUN3 device behind 5G-CRG |  |  |  |
| List of allowed combinations of GCI and HFC Node IDs | List of allowed combinations of GCI and HFC Node ID (NOTE 3) | Conditional (NOTE 1) | Yes | AM context |
| List of non-allowed combinations of GCI and HFC Node IDs | List of non-allowed combinations of GCI and HFC Node ID (NOTE 3). | Conditional (NOTE 1) | Yes | AM context |
| NOTE 1: If service area restrictions is enabled. NOTE 2: Either the list of allowed (GLIs or HFC Node IDs) or the list of non-allowed (GLIs or HFC Node IDs) are provided by the PCF. NOTE 3: Either the list of allowed (GCI and HFC Node ID combinations) or the list of non-allowed (GCI or HFC Node ID combinations) are provided by the PCF. |  |  |  |  |

The list of allowed GLIs or the list of allowed HFC Node IDs indicates the locations where the 5G-RG is allowed to be registered, see clause 4.3.3.3 for the description on how AMF uses this information.

# * * * * Second change (Issue #1) * * * *

### 8.1.1	Nudm_SubscriberDataManagement (SDM) Service

#### 8.1.1.1	General

In addition to the Subscription data types used in the Nudm_SubscriberDataManagement Service, as defined in Table 5.2.3.3.1-1 of TS 23.502 [3], the additional data types defined in Table 8.1.1.1-1 below are applicable for RGs connected to 5GC via W-5GAN and AUN3 devices.

Table 8.1.1.1-1: Wireline access specific UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| Access and Mobility Subscription data (data needed for UE Registration and Mobility Management) | RG Level Wireline Access Characteristics | The RG level Wireline Access Characteristics parameter provides QoS information for the W-AGF, as defined in clause 4.5.1.2. This parameter is handled by the UDM as a transparent container. This parameter may also be provisioned in subscriptions for AUN3 devices. |
|  | AUN3 device connectivity allowed. | Indicates whether the subscriber is allowed to access as an AUN3 device. Only provisioned in subscriptions for AUN3 devices. |

In the case of Wireline access, the Forbidden area information within Table 5.2.3.3.1-1 of TS 23.502 [3] may correspond to a (set of) allowed Global Line ID.

# * * * Third Change (Issue #2) * * * *

#### 7.2.8.3	5G-RG Deregistration via W-5GAN when it is also serving AUN3 devices

Figure 7.2.8.3-1: 5G-RG Deregistration procedure via W-5GAN when serving AUN3 devices

1-3.	The same as steps 1-3 for Figure 7.2.1-1 with the following modification:

In the case of 5G-RG initiated deregistration, 5G-RG shall first deregister each of the registered AUN3 devices connected to it (if any) before initiating the deregistration of itself.

4.	W-AGF controls if there exist any AUN3 devices connected to the 5G-RG that are registered to the 5GC through the W-AGF. If there are no AUN3 devices, then steps 5 and 6 shall be ignored.

5.	[conditional] For each AUN3 device identified in step 4, the W-AGF shall send AN Release request as specified in clause 4.2.6 of TS 23.502 [3]. Here, the cause should indicate the disconnection of 5G-RG.

6.	[conditional] Upon receiving the AN Release request with the cause specified in step 5, the AMF shall initiate deregistration of the AUN3 device as specified in clause 4.2.2.3.3 of TS 23.502 [3]. The cause in the deregistration message should indicate the disconnection of 5G-RG.

7.	The W-AGF may initiate the release of the signalling connection between 5G-RG and W-AGF.

NOTE:	Whether this step is needed, and if so, the details of this step are defined by BBF and/or CableLabs.

8.	The RG disconnects the AUN3 device. How this is done is outside of 3GPP scope.

# * * * * End of changes * * * *