SA WG2 Meeting #S2-160AH-e	S2-2400009

22 - 29 January, 2024, Electronic meeting

SA WG2 Meeting #S2-160	S2-2311951

13 - 17 November, 2023, Chicago, USA

3GPP TSG-CT WG1 Meeting #144	C1-237897

# Xiamen, China, 09– 13 October 2023

# Title:	LS on 5G ProSe UE-to-UE relay discovery with security aspects

# Release:	Rel-18

# Work Item:	5G_ProSe_Ph2

Source:	CT1

To:	SA2

Cc:	SA3

Contact Person:

Name:	Xiaoyan Duan

Tel. Number:

E-mail Address:	duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

CT1 is working on stage 3 procedures for 5G ProSe UE-to-UE relay discovery taking into account of the security requirements from SA3 (see TS 33.503 clause 6.1.3.3). Considering the impacts of the security requirements on the current 5G ProSe UE-to-UE relay discovery procedures as specified by SA2, CT1 would like to check with SA2 on the following aspect:

- As specified in TS 33.503 clause 6.1.3.3.3.2, for 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE.

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

2. Actions:

To SA2 group.

ACTION 1: CT1 kindly asks SA2 to provide answer to the above question.

3. Date of Next CT1 Meetings:

CT1#145	13th   – 17th November 2023 		Chicago