3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400365

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Tencent, Tencent Cloud

Title:	KI#2, New Sol: Support of Store and Forward Satellite Operation via Control Plane Buffering

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: This pCR proposes the solution to support store and forward satellite operation.

1. Introduction/Discussion

In Key Issue#2 of FS_5GSAT_ARCH_Ph3, the following aspects are captured:

5.2.1 Description

S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS). To support S&F Satellite operation for such services, it is proposed to study the following items:

- If applicable, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite(s) for the intended service(s);

- Whether and how to trigger S&F Satellite operation, and how to execute S&F Satellite operation;

- What enhancements on the related UE and network procedures are needed to support S&F Satellite operation, including:

- Whether to inform the UE when the S&F Satellite operation is applied or not.

NOTE 1: S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

NOTE 2: Coordination with SA3-LI on LI aspects is needed.

NOTE 3: The candidate solutions need to indicate which service(s) they are addressing (CIoT CP Optimizations, CIoT UP Optimizations, SMS).

This pCR proposes a new solution to support store and forward operation with control plane buffering.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change, all text new * * * *

## 6.X	Solution #X: Support of Store and Forward Operation with Control Plane Buffering

### 6.X.1	Key Issue mapping

This solution applies to Key Issue #2: support of store and forward satellite operation.

### 6.X.2	Description

To support store and forward satellite operation, this solution proposes new NF i.e. store and forward function (SFF) which collaborate the control plane NF i.e. SMF and NEF to store the uplink or downlink data.  The new NF can be standalone or collocated together with other control plane NF.

### 6.X.3	Procedures

Figure 6.X-1 Procedure of store and forward for uplink data

- UE setup control plane path and send uplink data to SMF

- SMF sends the data to SFF and indicate the time for storage

- SFF stores the data according to SMF instruction

- SFF sends the uplink data to SMF

- SMF sends the uplink data to AF via NEF

- SMF sends the uplink data to AF directly (alternative way for step 5)

Figure 6.X-2 Procedure of store and forward for downlink data

- SFF receives the downlink data from AF directly or via NEF

- SFF stores the downlink data for certain time

- SFF forwards the data to SMF

- SMF sends the downlink data to UE via control plane path e.g. NAS connection

### 6.X.4	Impacts on services, entities and interfaces

SFF:

- New control plane NF to support store and forward for uplink and downlink user data

SMF:

-   Obtain the stored user data and forward to UE via control plane NAS connection

# * * * * Second change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #2> |  |
| #X | x |  |

# * * * * End of changes * * * *