SA WG2 Meeting #160-Ad Hoc-e	S2-2400505

# Electronic meeting, January 22 – 29, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0197 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Tx Profile for SL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the LS from RAN2 on TX Profile for SL CA (S2-2400052 /R2-2313622), RAN2 informed their agreement: Tx profile was agreed to be introduced to solve the backward compatible issue for broadcast and groupcast of SL CA. RAN2 discussed the granularity of Tx profile and agreed that upper layer should indicate the Tx profile per QoS flow to AS layer. In this CR, it is proposed to introduce a new Tx profile to support this backward compatible issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An extended Tx profile is introduced to indicate whether the service supports SL CA. V2X layer provides the Tx profile to the AS layer per QoS flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The granularity is not aligned with RAN2 agreement. Backwards incompatible issue will be cause if the service is used for the non supporting UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.2.1, 5.4.1.1.3, 5.11, 6.3.1, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Application Layer connection-less group: An application layer group without group formation in the V2X application layer, e.g., sensor sharing.

Application Layer ID: An identifier identifying an entity, e.g. a vehicle, a pedestrian, an RSU within the context of a specific V2X application. The format of this identifier is outside the scope of 3GPP.

NOTE 1:	The Application Layer ID could be e.g. Station ID or Vehicle ID defined by other SDOs e.g. ETSI, Society of Automotive Engineers (SAE), etc.

NOTE 2:	The usage of Application Layer ID, e.g. one Application Layer ID is associated with one V2X application, one Application Layer ID is associated with more than one V2X applications, or one Application Layer ID is used for all V2X applications in the UE, is up to application layer implementation.

Application Layer managed group: An application layer group with group formation and management in the V2X application layer, e.g., platooning, cooperative adaptive cruise control.

Groupcast mode communication: It refers to V2X communication over PC5 reference point within a group of UEs where any UE in the group can act as transmitting UE and the rest act as receiving UEs. See also TS 38.300 [11] for more information. The "group" here refers to Application Layer managed group or Application Layer connection-less group.

Member ID: An identifier uniquely identifying a member in the Application Layer managed group and that is managed by the V2X application layer.

Mode of communication: Mode of communication to be used by the UE over PC5 reference point i.e. broadcast mode (LTE PC5 and NR PC5), groupcast mode (NR PC5) or unicast mode (NR PC5).

NR Tx Profile: The transmission mechanism or format to be used by the UE over NR PC5 RAT (e.g. PC5 DRX support or not), see TS 38.300 [11] and TS 38.331 [15].

NR Tx Profile for backwards compatiblity for PC5 CA operations in the AS layer: The backwards compatible for CA operations in the AS layer transmission mechanism or format to be used by the UE over NR PC5 RAT (e.g. backwards compatible or not), see TS 38.300 [11] and TS 38.331 [15].

V2X application: An application using one or more V2X services e.g. an active safety application in a vehicle, using V2X services such as emergency warning and vehicle to vehicle safety and awareness. A V2X application may operate towards a V2X Application Server.

V2X communication: A communication to support Vehicle-to-Everything (V2X) services leveraging Uu and / or PC5 reference points. V2X services are realized by various types of V2X applications, i.e. Vehicle-to-Vehicle (V2V), Vehicle-to-Pedestrian (V2P), Vehicle-to-Infrastructure (V2I) and Vehicle-to-Network (V2N).

V2X message: A dedicated messaging type of V2X service, for example ITS messages.

V2X service: A data service, offered to V2X applications and optionally V2X Application Servers. A V2X service belongs to one V2X service type. It may include message or other data delivery, as defined in TS 22.185 [2] and TS 22.186 [3]. A V2X service can be associated with one or more V2X applications, and a V2X application can be associated with one or more V2X services.

V2X service type: A type of V2X service, which is identified by e.g. ITS-AID (ITS Application Identifier), PSID (Provider Service Identifier) or AID (Application Identifier).

For the purposes of the present document, the following terms and definitions given in ISO 17419:2018 [4] apply:

Intelligent Transport Systems

ITS Application Identifier

For the purposes of the present document, the following term and definition given in IEEE Std 1609.12-2016 [5] apply:

Provider Service Identifier

For the purposes of the present document, the following term and definition given in CCSA YD/T 3707-2020 [27] applies:

Application Identifier

For the purposes of the present document, the following term and definition given in TS 23.285 [8] apply:

Tx Profile

* * * * Next Change * * * *

5.1.2.1	Policy/Parameter provisioning

The following sets of information for V2X communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by E-UTRA" or "served by NR":

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point when "served by E-UTRA" or "served by NR".

For each above PLMN:

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRA" and "not served by NR":

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR".

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

NOTE 1:	In this specification, {When the UE is "served by E-UTRA" or "served by NR"} and {When the UE is "not served by E-UTRA" and "not served by NR"} are relevant to V2X communications over PC5 reference point.

2)	Radio parameters when the UE is "not served by E-UTRA" and "not served by NR":

-	Includes the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". These radio parameters (e.g., frequency bands) are defined in TS 36.331 [14] and TS 38.331 [15]. The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

3)	Policy/parameters for PC5 RAT selection and for PC5 Tx Profile selection:

-	the mapping of V2X service types to PC5 RAT(s) (e.g. LTE PC5, NR PC5 or both), and:

-	for LTE PC5, to the corresponding Tx Profiles (see TS 36.300 [9] for further information);

-	for NR PC5, to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [11] and TS 38.331 [15] for further information);

-	for NR PC5, to the corresponding NR Tx Profiles for backwards compatiblity for CA operations in the AS layer for broadcast and groupcast (see TS 38.300 [11] and TS 38.331 [15] for further information).

NOTE 3:	Configuration related to backward compatibility requirements for V2X service types in the NR Tx Profiles for CA operations in the AS layer for broadcast and groupcast can be set per geographical area.

-	for NR PC5, to the corresponding NR Tx Profiles for transmitting and receiving initial signalling to establish unicast connection (see TS 38.300 [11] and TS 38.331 [15] for further information).

NOTE 4:	The policy/parameter ensures that the UEs of prior Release do not use V2X service types with a mapping of NR Tx Profile indicating the transmission mechanism or format specified in this Release.

NOTE 4a:	The policy/parameter ensures that the UEs of prior Release do not use V2X service types with a mapping of NR Tx Profile for backwards compatiblity for CA operations in the AS layer transmission mechanism or format specified in this Release.

4)	Policy/parameters related to privacy:

-	The list of V2X service types, with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when LTE PC5 is selected:

Same as specified in clause 4.4.1.1.2, item 3) of TS 23.285 [8] Policy/parameters except for the mapping of V2X service types to Tx Profiles and the list of V2X services with Geographical Area(s) that require privacy support.

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of V2X service types to V2X frequencies with Geographical Area(s).

-	The mapping of V2X service types to the default mode of communication (i.e. broadcast mode, groupcast mode or unicast mode).

-	The mapping of V2X service types to Destination Layer-2 ID(s) for broadcast.

-	The mapping of V2X service types to Destination Layer-2 ID(s) for groupcast mode communication.

-	The mapping of V2X service types to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 5:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one V2X service types. In the case where different V2X services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one V2X service types, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	The mapping of V2X service types to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	AS layer configurations (see TS 38.331 [15]), e.g. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by E-UTRA" and "not served by NR".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2, and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1.

-	For broadcast, groupcast and initial signalling to establish unicast connection, PC5 DRX configurations (see TS 38.331 [15]), e.g. the mapping of PC5 QoS profile(s) to PC5 DRX cycle(s), default PC5 DRX configuration, when the UE is "not served by E-UTRA" and "not served by NR".

7)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

The above parameter sets from bullet 2) to 6) may be configured in the UE through the V1 reference point by the V2X Application Server.

* * * * Next Change * * * *

5.4.1.1.3	Handling of PC5 QoS Flows based on PC5 QoS Rules

For each communication mode (e.g. broadcast, groupcast, unicast), the UE maintains the PC5 QoS Context and PC5 QoS Rule(s) for each PC5 QoS Flow identified by a PC5 QoS Flow Identifier (PFI) per destination identified by Destination Layer-2 ID.

The following information is maintained in the V2X layer of the UE:

-	A PC5 QoS Context includes the following information:

-	PFI;

-	PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, Range, etc.) as defined in clause 5.4.2; and

-	the V2X service type(s).

-	One or more PC5 QoS Rule(s). Each PC5 QoS Rule contains the following information:

-	PFI;

-	a PC5 Packet Filter Set as defined in clause 5.4.1.1.4; and

-	a precedence value. The precedence value determines the order in which the PC5 QoS Rules are evaluated. The PC5 QoS Rule with lower precedence value is evaluated before those with the higher precedence values.

NOTE:	How to set the precedence value is up to UE implementation.

When the UE assigns a new PFI for V2X service type, the UE stores it with the corresponding PC5 QoS Context and PC5 QoS Rule(s) for the destination. When the UE releases the PFI, the UE removes the corresponding PC5 QoS Context and PC5 QoS Rule(s) for the destination. For unicast, the Unicast Link Profile defined in clause 5.2.1.4 contains additional information mapped from PFI for unicast operation.

The V2X layer provides information for PC5 QoS operations per destination (e.g. identified by Destination Layer-2 ID) to AS layer for Per-flow QoS model operations as below:

1)	To add a new PC5 QoS Flow or to modify any existing PC5 QoS Flow, the V2X layer provides the following information for the PC5 QoS Flow to AS layer.

-	the PFI;

-	the corresponding PC5 QoS parameters; and

-	source/destination Layer-2 IDs for broadcast and groupcast mode communication, or the PC5 Link Identifier for unicast.

2)	To remove any existing PC5 QoS Flow, the V2X layer provides the following information for the PC5 QoS Flow to AS layer.

-	the PFI; and

-	source/destination Layer-2 IDs for broadcast and groupcast mode communication, or the PC5 Link Identifier for unicast.

In addition, the V2X layer also provides the communication mode (e.g. broadcast, groupcast, unicast) and radio frequencies for the PC5 QoS Flow to the AS layer for the PC5 operation. The radio frequencies are determined based on the V2X service type associated with the PC5 QoS Flow based on the configuration as described in clause 5.1.2.1. For broadcast or groupcast, the V2X layer also provides the NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer for the PC5 QoS Flow to the AS layer as specified in clauses 6.3.1 and 6.3.2. The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows. The V2X layer ensures that V2X service types associated with or without NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer are classified into distinct PC5 QoS Flows.

Figure 5.4.1.1.3-1 illustrated an example of the classification and marking of user plane traffic using the PC5 QoS Rules, and the mapping of PC5 QoS Flows to radio resources at access stratum layer.

Figure 5.4.1.1.3-1: Handling of PC5 QoS Flows based on PC5 QoS Rules

As illustrated in Figure 5.4.1.1.3-1, for a given pair of source and destination Layer-2 IDs, there can be multiple radio bearers, each corresponding to a different PC5 QoS level. The AS layer can determine the mapping of multiple PC5 QoS Flows to the same radio bearer based on the information provided. For broadcast and groupcast mode communication, the L2 link goes to all UEs in proximity identified by the destination Layer-2 ID.

* * * * Next Change * * * *

5.11	Support of NR PC5 Carrier Aggregation operations

For NR based unicast, groupcast and broadcast mode communication over PC5 reference point, PC5 Carrier Aggregation (CA) operations are supported.

Support for NR PC5 CA operations in the AS layer is specified in TS 38.300 [11].

* * * * Next Change * * * *

6.3.1	Broadcast mode V2X communication over PC5 reference point

To perform V2X communication over PC5 reference point in broadcast mode operation, the UE is configured with the related information as described in clause 5.1.2.

Figure 6.3.1-1 shows the procedure for broadcast mode of V2X communication over PC5 reference point.

Figure 6.3.1-1: Procedure for Broadcast mode of V2X communication over PC5 reference point

1.	The V2X layer of receiving UE(s) determines the following for the broadcast mode communication reception:

-	the destination Layer-2 ID for broadcast reception as specified in clause 5.6.1.2;

-	the PC5 QoS parameters for this broadcast V2X service as specified in clause 5.4.1.1;

-	the NR Tx Profile and NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer based on the configuration as specified in clause 5.1.2.1; and

-	the radio frequency information based on the configuration as specified in clause 5.1.2.1.

The destination Layer-2 ID, the NR Tx Profile and the PC5 QoS parameters are passed down to the AS layer of receiving UE(s) for the reception. When there is no NR Tx Profile or no NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer available to be mapped for this broadcast V2X service, the V2X layer does not provide NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer to the AS layer. When there are the other V2X service type(s) that use the same destination Layer-2 ID with this broadcast V2X service, the V2X layer determines whether and how to provide the NR Tx Profile to the AS layer as specified in clause 5.9.

The AS layer of receiving UE(s) determines the PC5 DRX parameter values as specified in clause 5.9.

The radio frequency information for this broadcast V2X service is also passed down to the AS layer of receiving UE(s) for the reception.

2.	The transmitting UE V2X application layer provides data unit and may provide V2X Application Requirements specified in clause 5.4.1.2 to V2X layer.

3.	The V2X layer of transmitting UE determines the following for the broadcast mode communication transmission:

-	the destination Layer-2 ID for broadcast as specified in clause 5.6.1.2;

-	the PC5 QoS parameters for this broadcast V2X service as specified in clauses 5.4.1.1 and 5.4.1.2;

-	the NR Tx Profile and NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer based on the configuration as specified in clause 5.1.2.1; and

-	the radio frequency information based on the configuration as specified in clause 5.1.2.1.

The transmitting UE self-assigns the source Layer-2 ID as specified in clause 5.6.1.1.

The source Layer-2 ID, the destination Layer-2 ID, the NR Tx Profile, NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer and the PC5 QoS parameters are passed down to the AS layer of transmitting UE for the transmission. When there is no NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer available to be mapped for this broadcast V2X service, the V2X layer does not provide NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer to the AS layer. When there are the other V2X service type(s) that use the same destination Layer-2 ID with this broadcast V2X service, the V2X layer determines whether and how to provide the NR Tx Profile to the AS layer as specified in clause 5.9.

The AS layer of transmitting UE determines the PC5 DRX parameter values as specified in clause 5.9.

The radio frequency information for this broadcast V2X service is also passed down to the AS layer of transmitting UE for the transmission.

4.	The transmitting UE sends the V2X service data using the source Layer-2 ID and the destination Layer-2 ID.

NOTE:	In step 4, there is only one broadcast message from the transmitting UE.

* * * * Next Change * * * *

6.3.2	Groupcast mode V2X communication over PC5 reference point

To perform groupcast mode of V2X communication over PC5 reference point, the UE is configured with the related information as described in clause 5.1.2.1.

Figure 6.3.2-1 shows the procedure for groupcast mode of V2X communication over PC5 reference point.

Figure 6.3.2-1: Procedure for groupcast mode of V2X communication over PC5 reference point

1.	V2X group management is carried out by the V2X application layer and is out of scope of this specification.

2.	The V2X application layer may provide group identifier information (i.e. an Application-layer V2X Group identifier) as specified in clause 5.6.1.3.

The V2X application layer may provide V2X Application Requirements for this communication.

If the V2X application layer does not provide V2X Application Requirements, the V2X layer determines the PC5 QoS parameters based on the mapping of V2X service type to PC5 QoS parameters as defined in clause 5.1.2.1.

The V2X application layer may provide a group size and a member ID as specified in clause 5.2.1.3.

3.	The V2X layer of transmitting UE determines the following for the groupcast mode communication transmission:

-	a source Layer-2 ID and a destination Layer-2 ID as specified in clauses 5.6.1.1 and 5.6.1.3;

-	the PC5 QoS parameters for this groupcast mode communication as specified in clauses 5.4.1.1 and 5.4.1.3;

-	the NR Tx Profile and NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer based on the configuration as specified in clause 5.1.2.1; and

-	the radio frequency information based on the configuration as specified in clause 5.1.2.1.

The source Layer-2 ID, destination Layer-2 ID, the NR Tx Profile, NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer and the PC5 QoS parameters are passed down to the AS layer of transmitting UE for the groupcast mode communication transmission. When there is no NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer available to be mapped for this groupcast V2X service, the V2X layer does not provide NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer to the AS layer. When there are the other V2X service type(s) that use the same destination Layer-2 ID with this groupcast V2X service, the V2X layer determines whether and how to provide the NR Tx Profile to the AS layer as specified in clause 5.9.

The radio frequency information for this groupcast V2X service is also passed down to the AS layer of transmitting UE for the transmission.

The V2X layer of receiving UE(s) determines the following for the groupcast mode communication reception:

-	the destination Layer-2 ID, as specified in clauses 5.6.1.1 and 5.6.1.3;

-	the PC5 QoS parameters for this groupcast mode communication as specified in clause 5.4.1.1;

-	the NR Tx Profile and NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer based on the configuration as specified in clause 5.1.2.1; and

-	the radio frequency information based on the configuration as specified in clause 5.1.2.1.

The destination Layer-2 ID, the NR Tx Profile, NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer and the PC5 QoS parameters are passed down to the AS layer of receiving UE(s) for the groupcast mode communication reception. When there is no NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer available to be mapped for this groupcast V2X service, the V2X layer does not provide NR Tx Profile or NR Tx Profiles for backwards compatiblity for PC5 CA operations in the AS layer to the AS layer. When there are the other V2X service type(s) that use the same destination Layer-2 ID with this groupcast V2X service, the V2X layer determines whether and how to provide the NR Tx Profile to the AS layer as specified in clause 5.9.

The radio frequency information for this groupcast V2X service is also passed down to the AS layer of receiving UE(s) for the reception.

If the group size and the member ID for Application Layer managed group are provided by the V2X application layer, the V2X layer passes them to the AS layer as described in clause 5.2.1.3.

The AS layer of transmitting UE and the AS layer of receiving UE(s) determine the PC5 DRX parameter values as specified in clause 5.9.

4.	Transmitting UE has a V2X service associated with this groupcast mode communication.

Transmitting UE sends the V2X service data using the source Layer-2 ID and the destination Layer-2 ID.

NOTE:	In step 4, there is only one groupcast message from the transmitting UE.

* * * * End of Changes * * * *