3GPP TSG-SA2 Meeting #160AHE	S2-2400525

# January 22 – 29, 2024, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4692 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE MSISDN exposure to trusted AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To use the localized services, the UE need to consider availability of time and location of localized services. In order to inform the UE of this information, the validity information is introduced. Therefore, introducing separated two parameters, validity information and location assistance information, is not reasonable. Even, we explicitly mentioned “the location assistance information and is not used for any area restriction enforcement.” in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification on AF specific UE ID retrieval procedures to support providng the MSISDN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution to providing the extern entities with the UE MSISDN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10, 5.2.6.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

4.15.10	UE GPSI retrieval

This clause contains the detailed description and the procedures for the UE GPSI retrieval which is either the AF specific UE ID (i.e. the External Identifier) or and the MSISDN as defined in TS 23.003 [33].

NOTE 1:	After retrieving AF specific UE ID, the AF can invoke NEF provided services (e.g. location monitoring).

Figure 4.15.10-1: UE GPSI retrieval

1.	AF requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message shall include UE ID TYPE (either AF specific UE ID, MSISDN or both), UE address (IP address or MAC address) and AF Identifier, it may include, Port Number associated with the IP address, MTC Provider Information, Application Port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application. If available, the AF may also provide the corresponding DNN and/or S-NSSAI.

NOTE 2:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

NOTE 3:	The combination of IP address and Port Number can be used by 5GC to derive the UE private IP address assigned by 5GC if the UE is behind a NAT, see steps 3-6 below.

NOTE 4:	The Application Port ID is as defined in Nnef_Trigger_Delivery.

NOTE 5:	The NEF can validate the provided MTC Provider Information and override it to a NEF selected MTC Provider Information based on configuration. How the NEF determines the MTC Provider Information, if not present, is left to implementation (e.g. based on the requesting AF).

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps. The NEF determines corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or is determined by the NEF based on the requesting AF Identifier, MTC Provider Information.

If the NEF has received a Port Number in step 1, based on configuration, the NEF may recognize the address received is an IP address which is different from the actual private UE IP address assigned by 5GC, i.e. the UE is behind a NAT in UPF. If so, the NEF performs steps 3 to 6. Otherwise, steps 3 to 6 are skipped.

3.	The NEF uses the Nnrf_NFDiscovery service operation to obtain the address of the UPF implementing NAT functionality for the UE (public) IP address. The request includes the UE (public) IP address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

4.	The NRF responds with a Nnrf_NFDiscovery response message including the UPF address of the UPF implementing NAT functionality for the UE (public) IP address.

5.	The NEF uses the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get service operation to request UE's (private) IP address from the UPF. The request includes the UE (public) IP address and Port Number and optionally IP domain, DNN and S-NSSAI associated with the AF ID.

6.	The UPF responds with the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get response message including UE's IP address and optionally, the IP domain. If the UPF has applied a NAT functionality, the UE's IP address returned by the UPF is the private UE IP address. If IP domain of UE private IP address is returned from UPF, it always takes precedence regardless of whether the IP domain information also provided by AF when it invokes Nnef_UEId_Get service operation. If the AF requests to retrieve the AF specific UE ID and the UPF has the SUPI or MSISDN of the UE, the UPF may return SUPI or MSISDN and in this case steps 7-8 are skipped. If the AF requests to retrieve the MSISDN and the UPF has the SUPI of the UE, the UPF may return the SUPI and in this case steps 7-8 are skipped. If the AF requests to retrieve the MSISDN and the UPF has the MSISDN of the UE, the UPF may return the MSISDN and in this case steps 7-10 are skipped.

NOTE 6:	The SUPI/MSISDN is only available when the SMF provides it to the UPF for the purposes defined in TS 29.244 [69].

7-8.	The NEF uses the Nbsf_Management_Discovery service operation with UE address and IP domain and /or DNN and/or S-NSSAI to retrieve the session binding information of the UE. If no SUPI is received in the session binding information from the BSF, the NEF replies to the AF with a Result value indicating that the UE ID is not available.

9.	The NEF interacts with UDM to retrieve the GPSI via the Nudm_SDM_Get service operation. The request message includes SUPI or MSISDN and at least one of Application Port ID, MTC Provider Information or AF Identifier.

10.	The UDM responds to the NEF with the GPSI for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier.

11.	The NEF further responds to the AF with the information (including the GPSI) received from the UDM.

* * * Second Changes * * *

5.2.6.27	Nnef_UEId service

5.2.6.27.1	General

See clause 4.15.10 and clause 4.3.6.5.

5.2.6.27.2	Nnef_UEId_Get operation

Service operation name: Nnef_UEId_Get

Description: Get the UE identifier.

Inputs, Required: UE ID Type (i.e. either AF specific UE identifier, MSISDN or both), GPSI or UE address (i.e. IPv4/IPv6 address or MAC address) or External Group Identifier(s).

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information, AF Identifier.

Outputs, Required: Result, AF specific UE Identifier represented as an External Identifier or SUPI or Internal Group Identifier(s) or MSISDN.

NOTE:	SUPI and Internal Group Identifier can only be exposed to roaming partners.

Outputs, Optional: None.

* * * End of Changes * * *