3GPP TSG-WG SA2 Meeting #160AHE	S2-2400602

January 22 - 29, 2024, Electronic Meeting	(revision of S2-24xxxxx)

Source:	CATT

Title:	Key Issue for WT#1.2: Enhancement of Registration Procedure for DualSteer Devices

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new KI based on the approved WT#1.2.

1. Discussion

The WT#1.2 of FS_MASSS SID is described as following:

WT#1: Study the overall architecture and function enhancements to 5GS to support a DualSteer Device (see TS 22.261 for definition of DualSteer Device). A DualSteer Device supports traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be (a) a single UE, in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs in case of simultaneous data transmission over the two networks. The subscriber of the DualSteer Device has two subscriptions/SUPIs, sharing one subscription profile from the same operator. For any particular service, at any given time, the DualSteer Device shall transmit all traffic of that service using only a single 3GPP access network.

The following scenarios are considered:

1. Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4. NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

NOTE 2: 	The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

NOTE 3:	For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

NOTE 4:	The 5GC-EPC scenarios will be studied once the baseline 5GC-5GC scenarios are stable.

NOTE 5:	Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

WT#1.2: Study whether there are any impacts to the registration procedure for DualSteer Devices.

NOTE 6:	This WT requires coordination with SA3 for handling security aspects.

NOTE 7:	No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

NOTE 8:	If any PLMN selection enhancement is identified, SA2’s role could be limited to study system level impacts and/or trigger SA1 and CT1 to handle the work.

Based on the approved WT#1.2, it is proposed to do the following study:

-	Study whether and how to enhance registration procedure for DualSteer Device;

2. Proposal

It is proposed to add the following changes to the TR 23.700-54 V0.0.0.

# * * * * First change * * * *

5.X	Key Issue #<X>: Enhancement of registration procedure for DualSteer Devices

5.X.1	Description

The DualSteer Device can be a single UE in case of non-simultaneous data transmission over the two networks or two separate UEs in case of simultaneous data transmission over the two networks.

To investigate how the DualSteer Device acting as s single UE or two separate UEs triggers the registration procedure separately, the following items should be studied:

-	Study whether and how to enhance the registration procedure for DualSteer Device;

NOTE 1:	This work on this key issue may need coordination with SA3 for handling security aspects.

NOTE 2:	This work on this key issue does not impact on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

NOTE 3:	Any PLMN selection enhancement on this key issue may need coordination with SA1 and CT1.

# * * * * End of Change * * * *