SA WG2 Meeting #160-AHE	S2-2400604

January 22 - 29, 2024, Electronic Meeting

Source:	CATT

Title:	Architectural Assumptions and Requirements for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes architectural assumptions and requirements for DualSteer.

1	Discussion

This paper proposes an architectural assumptions and requirements for DualSteer based on the output of the FS_MASSS SID and the DualSteer requirements in TS 22.261.

The following is some of contents included in the objectives of the FS_MASSS SID:

- The following scenarios are considered:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

- The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

- For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

- Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

- No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

The service requirements for Traffic steering and switching over two 3GPP access networks has been defined in clause 6.50 of TS 22.261, which includes the following aspects:

- Scenarios may also include traffic steering and/or switching across LTE/EPC and NR/5GC, with anchoring in 5GC.

- Traffic policies are intended to be in full control of the home network operator.

- The requirements can apply to different DualSteer device types (e.g., smartphones, IoT, UAV, VSAT devices).

- The requirements apply to a subscriber with two subscriptions/SUPIs, sharing one subscription profile from the same operator. For simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

- Inter-PLMN requirements can apply also to PLMN-NPN scenarios assuming a PLMN-integrated NPN (NPN hosted by a PLMN or offered as a slice of a PLMN).

- For any particular service, at any given time, the DualSteer device shall transmit all traffic of that service using only a single 3GPP access network.

- Charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business/roaming agreement among network operators is assumed.

- The NPN is hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business/roaming agreement between the PLMN and the NPN operator (if different).

- The 5G system shall be able to support mechanisms to minimize service interruption when switching a DualSteer device’s user data, for one or multiple services, between two 3GPP access networks.

- The 5G system may be able to support mechanisms to change one 3GPP access network to the non-3GPP access network of the same subscription (and vice versa).

2	Proposal

It is proposed to include the following changes to TR 23.700-54 V0.0.0.

# * * * * First change * * * *

4.1.1	Architectural Assumptions for DualSteer

-	Assuming a subscriber has two subscriptions/SUPIs, sharing one subscription profile from the same operator.

-	For simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

-	For any particular service, at any given time, the DualSteer Device shall transmit all traffic of that service using only a single 3GPP access network.

-	Each subscription (i.e. SUPI) of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

-	Traffic policies are intended to be in full control of the home network operator.

-	If two 3GPP access networks belonging to two PLMNs, assuming a business/roaming agreement between PLMN operators (if different) and HPLMN data anchoring.

-	No coordination is assumed in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

-	No impacts are assumed for network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

# * * * * Next change * * * *

4.2.1	Architectural Requirements for DualSteer

- Support to differentiate the two connections for the same device and minimize impacts to CN, O&M or IT systems.

- Support mechanisms to minimize service interruption when switching a DualSteer device’s user data, for one or multiple services, between two 3GPP access networks.

- Support mechanisms to change one 3GPP access network to the non-3GPP access network of the same subscription (and vice versa).

- The requirements can apply to different DualSteer device types (e.g., smartphones, IoT, UAV, VSAT devices).

- When traffic steering and/or switching only across LTE/EPC and NR/5GC, the data is anchoring in 5GC.

- Support traffic steering and/or switching of a DualSteer device’s user data between a NPN and a PLMN, assuming non-simultaneous transmission over the two 3GPP access networks, a NPN hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business/roaming agreement between the PLMN and the NPN operator (if different).

- Support to collect charging information of user data across two 3GPP access networks.

- Support not to impact VPLMNs and/or HPLMNs that do not support traffic steering and/or switching of a DualSteer device.

# * * * * End of Change * * * *