SA WG2 Meeting #160-Ad Hoc-e	S2-2400625

E-meeting, January 22 – 29, 2024	(revision of S2-24xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0212 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling EAS IP address that not authorized in case of Local DNS for HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In section 6.7.2.2, if the HR-SBO is authorized, the H-SMF generates VPLMN Specific Offloading Information (i.e. IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN in VPLMN. Also, if the EAS Discovery Procedure with V-EASDF for HR-SBO, if VPLMN Specific Offloading Information does not include FQDN range and the EAS IP address of the DNS response is not part of the IP range(s) authorized, the V-SMF indicates the V-EASDF to do another DNS query. So, in conclusion, the V-SMF can judge whether the IP address in DNS response is authorized or not. But for EAS Discovery Procedure with Local DNS for HR-SBO, the DNS response will be directly sent to UE with no judgement by V-SMF. If the local DNS server resolves an IP address that not authorized by HPLMN, how to handle the case is unclear. One possible way is, if the V-UPF detects the traffic towards EAS IP address that not authorized to route to local, the V-UPF transmits the packet to H-UPF to further handling, for example, route the packet to EAS IP address via N6 interface in HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the local DNS server resolves an IP address that not authorized by HPLMN, the V-UPF delivers the packet to H-UPF to further routing to EAS IP address via N6 interface in HPLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the local DNS server resolves an IP address that not authorized by HPLMN, how to handle the case is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

6.7.2.4	EAS Discovery Procedure with Local DNS for HR-SBO

Figure 6.7.2.4-1: Procedure for EAS Discovery with local DNS for HR-SBO roaming scenario

The procedure in this clause assumes that the UL CL is able to detect FQDN(s) in traffic sent to the IP address of the local DNS Server. It is thus incompatible with usage of DoT (DNS over TLS) or DoH to protect the DNS traffic exchanged between the UE and the PLMN.

If the target FQDN of the DNS query is not part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1, the UL CL/BP UPF is instructed to send the DNS request to the DNS server address provided by HPLMN via V-UPF (if it exists) and H-UPF (through N9), by modifying the packet's destination IP address (corresponding to local DNS Server) to the DNS server address provided by HPLMN on UL CL or H-UPF . For the corresponding DNS response received by H-UPF, the H-UPF or UL CL modifies the packets' destination IP address to that of the local DNS Server.

If the EAS IP address is not part of the IP address authorized by H-SMF in step 2 of Figure 6.7.2.2-1, the UL CL/BP V-UPF is instructed to send the packet from UE to the H-UPF address provided by HPLMN via V-UPF (if it exists), by outer header creation (corresponding to H-UPF). And the H-UPF is instructed to remove the header and send the packet to EAS IP address to DN by N6 interface in HPLMN.

The steps 0 to 5 are the same as the steps 0 to 6 of Figure 6.2.3.2.3-1 with following differences:

-	SMF is replaced with V-SMF.

-	UE, (R)AN, AMF, UL CL/BP UPF, L-PSA UPF, V-SMF, Local DNS Resolver/Server are located in VPLMN.

-	UPF, H-SMF, C-DNS are located in HPLMN.

0.	The HR-SBO PDU Session is established. See the procedure in clause 6.7.2.2.

1.	UL CL/BP insertion. See the step 1 of the procedure in Figure 6.2.3.2.3.

2.	After UL CL/BP insertion is performed, the V-SMF sends new local DNS server address to the UE by performing PDU Session Modification procedure as in clause 4.3.3.3 of TS 23.502 [3] with following additions:

-	V-SMF sends Local DNS Server/Resolver to the H-SMF in the step 1a of the procedure as in clause 4.3.3.3 of TS 23.502 [3].

-	H-SMF sends the Local DNS Server/Resolver to be sent to the UE via PCO to the V-SMF in the step 3 of the procedure in clause 4.3.3.3 of TS 23.502 [3].

-	H-SMF instructs the address is not part of the authorization to V-SMF. If the packet is sent to the EAS IP address that not part of authorized by H-SMF, the UL CL/BP V-UPF is instructed by V-SMF to send the packet to the H-UPF address provided by HPLMN.

3-5.	See the steps 4-6 of the procedure in Figure 6.2.3.2.3.

* * * Start of Change 2 * * *