3GPP TSG-WG SA2 Meeting #160AHE 	S2-2400652

# 21 – 29, Jan. 2024, Elbonia	     		       (revision of S2-24xxxxx)

Source:	Google

Title:	Way forward proposal for NB-IoT UE support for coarse location reporting

Document for:	Discussion

Agenda Item:	9.2.2

Work Item / Release:	5GSAT_Ph2 / Rel-18

Abstract: This paper proposes way forwards for enabling NB-IoT UE support for coarse location reporting.

# 1. Discussion

In SA#102, SP-231594 and SP-231595 have been proposed to enable NB-IoT UE support for coarse location reporting over satellite access. The proposal motivation was:

- in technical perspective that, lacking of sufficiently accurate user location information (ULI) reported to the MME from the massive NB-IoT UEs located at IoT NTN coverage area across multiple regulatory areas would result in significant unnecessary signaling overheads. It’s because the MME would proceed with MM/SM procedures but eventually fail location verification or reject the MM/SM requests from the NB-IoT UE.

- in services perspectives, there are increasing interests from network operators, satellite service providers, and device vendors on enabling NB-IoT UE to report coarse location information when it accesses IoT NTN.

Observation#1: RRC reporting of UE Coarse Location is possible based on 3GPP TS36.331/38.331 using UEInformationRequest/UEInformationResponse messages for eMTC, NR UE using UECapabilityEnquiry/ UECapabilityInformation messages after successful security activation.

Proposal#1: RRC based coarse location reporting already supports for eMTC, NR UEs. For a new feature introduced in late R18 stage, it is proposed to leverage existing RRC based solution instead of inventing a new wheel (NAS based reporting of UE Coarse Location) specifically for NB-IoT UE. In addition, the approach needs to consider the least NB-IoT UEs impacts.

Observation#2: Currently, it is infeasible for an NB-IoT UE to report coarse location information over AS to an eNB due to limited bandwidth, and simplified radio design. This is because in general circumstances NB-IoT UE focus on low-power, low-throughput applications where exact location may not be a necessary requirement. If NB-IoT UE capability can afford of coarse location reporting, it is true that some of the limitation on NB-IoT UE on enabling coarse location reporting can be relaxed.

Obervation#3: the limitations is also due to security and privacy concerns from operators and regulators, which need to be resolved when considering to enable NB-IoT UE to report coarse location.

Proposal#2: Coarse location reporting should not be enabled for all NB-IoT UE, i.e. the network shall determine whether this feature can be enabled for a NB-IoT UE based on the following criteria:

- for a specific RAT, e.g. NB-IoT UE over satellite access

- NB-IoT UE capability support of location reporting

- network supports of location reporting

- security and privacy requirements from network operator and regulator

- NB-IoT UE subscription that may include user consent or indication for allowing coarse location reporting

Proposal#3: based on proposal#1 and proposal#2, it is proposed the following enhancement for network initiated RRC based coarse location reporting:

- Add UE subscription information: Allowed coarse location reporting indication that refers to user consent, requirement of operator and regulator

- Add MME’s functionality to store Allowed location reporting indication received from HSS in MME UE context, determine whether a NB-IoT UE is allowed for coarse location reporting, and request location reporting from eNodeB if the target NB-IoT UE is eligible for coarse location reporting

- Enhance procedures based on network initiated RRC based coarse location reporting.

- Attach procedure that MME can check and determine whether a NB-IoT UE is allowed for coarse location reporting

- Location reporting procedure that can support new request type for coarse location information.

# 2. Conclusion

It is proposed to agree on the above proposals as way forward and implement the corresponding changes in the related TS23.401 CR.