SA WG2 Meeting #160AH	S2-2400748

# E-meeting, January 22 – 29, 2024      		       		          (revision of S2-240)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New WID on Edge Computing for EPC Interworking

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	WID on Edge Computing for EPC Interworking

Acronym:	TEI19_EC4EPCIWK

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X(*) |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

(*) ME impacts are limited to the support over EPS of the same NAS PCO information related with Edge Computing than supported over 5GS

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Edge Computing is an important feature that can both improve end-user services (via shorter user plane delays) and lower operator transmission costs (via shorter in network data transmissions). It is nevertheless currently not available for EPC.

Many deployed networks have following characteristics:

- There is no ubiquitous 5GS coverage meaning that currently Edge Computing is not ubiquitously available in the PLMN,

- 5GC-EPC interworking is deployed with the possibility for UE(s) served by EPC to have their PDN connections controlled by a SMF+PGW-c that can receive PCC rules with Application Function influence on traffic routing Enforcement Control information (see TS 23.503 Table 6.3.1) determined based on Nnef-traffic-Influence API

Leveraging the deployed 5GC-EPC interworking to support Edge Computing for UE(s) served by EPC would be useful. To ensure a realistic deployment opportunity (considering that this WID is for R19) the solution impacts to EPC (MME and SGW) should be minimum (for example it is not desirable to replicate the complexity of the N16a interface between the anchor SMF and I-SMF in EPC).

# 4	Objective

Following aspects will be specified:

- Enabling the PGW-C to insert an UL Classifer (UL/CL) close to or co-located with SGW-U, and to change the UL/CL when the SGW-U changes (e.g. as a result of UE mobility), such as to be able to offload selective traffic. This includes:

- enhancing the S5 GTP-C procedures to support enabling the SGW-C to update the SMF+PGW-C with the SGW-U node name of the (new) SGW-U selected for the PDN connection.

- enable the SMF+PGW-C to discover and select an UL/CL (and local PSA) co-located with the SGW-U.

This enables running Edge Computing procedures controlled by the SMF+PGW-c (based on Application Function influence on traffic routing Enforcement Control information in PCC rules defined in TS 23.503 Table 6.3.1)

The resulting architecture is as follows:

Notes

- Even though this does not require protocol or procedure change, the SMF+PGW-c has to configure the UL CL to exchange GTP-u traffic with the SGW-u using EPC specifications i.e. with one GTP-u Tunnel per EPC bearer (per QoS class) and not using 5GC specifications i.e. with one GTP-u Tunnel per PDU Session (any QoS class) .

- When the SGW-u and the UL CL are co-located (combo node), this unique node is controlled by 2 PFCP Sessions: one from the SGW-c and one from the SMF+PGW-c;

The work will be limited to non-roaming cases

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | .75 |  |  |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| None |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.401 | Support of Edge Computing for EPC Interworking | SA#104 June 2024 |  |
| 23.502 | Support of Edge Computing for EPC Interworking | SA#104 June 2024 |  |
| 23.548 | Support of Edge Computing for EPC Interworking | SA#104 June 2024 |  |

# 6	Work item Rapporteur(s)

, Nokia, at nokia dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

OAM and charging aspects covered by SA5.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell