SA WG2 Meeting #160-Ad Hoc-e	S2-2400894

E-meeting, January 22 – 29, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1382 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on IMS DC Capability Negotiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause AC.7.0 of TS 23.228, for IMS DC capability negotiation a Feature-Caps header field is used to indicate data channel capability. It is not clearly stated the exact feature-capability indicator in this header field. In TS 24.186, the feature-capability indicator "g.3gpp.datachannel" is specified. It is proposed to add the reference to TS 24.186 in clause AC.8.1 of TS 23.338 to clarify in which specification the feature-capability indicator is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the reference to TS 24.186 in clause AC.8.1 of TS 23.338 to clarify the feature-capability indicator is specified in clause 9.2 of TS 24.186. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to indicate data channel capability using Feature-Caps header field if only reading TS 23.228. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, AC.7.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network Architecture".

[2]	CCITT Recommendation E.164: "Numbering plan for the ISDN era".

[3]	CCITT Recommendation Q.65: "Methodology – Stage 2 of the method for the characterisation of services supported by an ISDN".

[4]	ITU Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	Void.

[7]	3GPP TS 23.221: "Architectural Requirements".

[8]	3GPP TS 22.228: "Service requirements for the IP multimedia core network subsystem".

[9]	3GPP TS 23.207: "End-to-end QoS concept and architecture".

[10]	Void.

[10a]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[11]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[11a]	3GPP TS 29.207: "Policy control over Go interface".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	IETF RFC 3986: "Uniform Resource Identifiers (URI): Generic Syntax".

[14]	IETF RFC 4282: "The Network Access Identifier".

[15]	IETF RFC 3966: "The tel URI for Telephone Numbers".

[16]	IETF RFC 3761 (April 2004): "The E.164 to Uniform Resource Identifiers (URI) Dynamic Delegation Discovery System (DDDS) Application (ENUM)".

[16a]	IETF RFC 4941: "Privacy Extensions for Stateless Address Autoconfiguration in IPv6".

[17]	ITU Recommendation G.711: "Pulse code modulation (PCM) of voice frequencies".

[18]	ITU Recommendation H.248: "Gateway control protocol".

[19]	3GPP TS 33.203: "Access Security for IP-based services".

[20]	3GPP TS 33.210: "Network Domain Security: IP network layer security".

[21]	Void.

[22]	3GPP TR 22.941: "IP Based Multimedia Services Framework".

[23]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[24]	3GPP TS 23.003: "Technical Specification Group Core Network; Numbering, addressing and identification".

[25]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[26]	3GPP TS 32.260: "Telecommunication Management; Charging Management; IP Multimedia Subsystem (IMS) charging".

[27]	3GPP TS 22.071: "Technical Specification Group Services and System Aspects, Location Services (LCS); Service description, Stage 1".

[28]	3GPP TS 23.271: "Technical Specification Group Services and System Aspects, Functional stage 2 description of LCS".

[29]	3GPP TS 23.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 3 - Stage 2".

[29a]	3GPP TS 22.340: "IMS Messaging; Stage 1".

[30]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[31]	3GPP TS 23.240: "3GPP Generic User Profile - Architecture; Stage 2".

[32]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[33]	IETF RFC 2766: "Network Address Translation-Protocol Translation (NAT-PT)".

[34]	IETF RFC 2663: "IP Network Address Translator (NAT) Terminology and Considerations".

[35]	Void.

[36]	3GPP TS 23.141: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Presence Service".

[37]	Void.

[38]	IETF RFC 3840: "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[39]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[40]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Network".

[41]	IETF RFC 3312: "Integration of resource management and Session Initiation Protocol (SIP)".

[42]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[43]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[44]	IETF RFC 3263: "Session Initiation Protocol (SIP): Locating SIP Servers".

[45]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[46]	IETF RFC 5766: "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[47]	IETF RFC 5389: "Session Traversal Utilities for NAT (STUN)".

[48]	IETF RFC 5626: "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[49]	IETF RFC 5627: "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[50]	IETF RFC 5628: "Registration Event Package Extension for Session Initiation Protocol (SIP) Globally Routable User Agent URIs (GRUUs)".

[51]	IETF RFC 4787: "Network Address Translation (NAT) Behavioural Requirements for Unicast UDP".

[52]	3GPP TS 23.279: "Combining Circuit Switched (CS) and IP Multimedia Subsystem (IMS) services; Stage 2".

[53]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[54]	3GPP TS 23.203: "Policy and Charging Control architecture".

[55]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[56]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access".

[57]	IETF RFC 4769: "IANA Registration for an Enumservice Containing Public Switched Telephone Network (PSTN) Signaling Information".

[58]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[59]	3GPP TS 29.333: "Multimedia Resource Function Controller (MRFC) - Multimedia Resource Function Processor (MRFP) Mp Interface; Stage 3".

[60]	3GPP2 X.S0011: "cdma2000 Wireless IP Network Standard".

[61]	3GPP2 C.S0001-D: "Introduction to cdma2000 Spread Spectrum Systems - Revision D".

[62]	3GPP2 C.S0024-A: "cdma2000 High Rate Packet Data Air Interface Standard, April 2004".

[63]	3GPP2 C.S0084-000: "Overview for Ultra Mobile Broadband (UMB) Air Interface Specification".

[64]	3GPP TS 24.167: "3GPP IMS Management Object (MO); Stage 3".

[65]	IETF RFC 3022: "Traditional IP Network Address Translator (Traditional NAT)".

[66]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services".

[67]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity".

[68]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[69]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[70]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[71]	3GPP TS 23.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[72]	IETF RFC 3264: "An Offer/Answer Model with Session Description Protocol".

[73]	3GPP TS 23.333: "Multimedia Resource Function Controller (MRFC) - Multimedia Resource Function Processor (MRFP) Mp interface: Procedures Descriptions".

[74]	3GPP TS 23.334: " IMS Application Level Gateway (IMS-ALG) - IMS Access Gateway (IMS-AGW) interface: Procedures Descriptions".

[75]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks".

[76]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[77]	3GPP TS 22.153: "Multimedia priority service".

[78]	ETSI ES 282 003: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control Sub-System (RACS): Functional Architecture".

[79]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh Interface; Signalling flows and message contents".

[80]	3GPP TS 23.380: "IP Multimedia Subsystem (IMS); IMS Restoration Procedures".

[81]	3GPP TS 24.525: "Business trunking; Architecture and functional description".

[82]	3GPP TS 23.402: "Architecture Enhancements for non-3GPP accesses".

[83]	3GPP TS 33.328: "IP Multimedia (IM) Subsystem media plane security".

[84]	IETF RFC 8825: "Overview: Real Time Protocols for Brower-based Applications".

[85]	W3C: "WebRTC 1.0: Real-time Communication Between Browsers", W3C Recommendation, 26 January 2021, https://www.w3.org/TR/2021/REC-webrtc-20210126/.

[86]	W3C: "Cross-Origin Resource Sharing", W3C Proposed Recommendation, 05 December 2013, http://www.w3.org/TR/2013/PR-cors-20131205/.

[87]	ITU-T Recommendation T.140: "Protocol for multimedia application text conversation".

[88]	IETF RFC 6455: "The WebSocket Protocol".

[89]	IETF RFC 7118: "The WebSocket Protocol as a Transport for the Session Initiation Protocol (SIP)".

[90]	IETF RFC 4571: "Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport".

[91]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[92]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[93]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[94]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[95]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[96]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[97]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[98]	3GPP TS 29.563: "5G System (5GS); Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[99]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[100]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[101]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[102]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[103]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[104]	3GPP TS 26.264: "IMS-based AR Real-Time Communication".

[X]	3GPP TS 24.186: "IMS Data Channel applications; Protocol specification".

SECOND CHANGE

AC.7.0	IMS DC capability negotiation

The IMS network and the UE need to mutually negotiate the capability of supporting IMS data channel when a UE supporting IMS data channel registers to IMS network.

IMS data channel capability negotiation includes two aspects:

-	The network discovers the data channel capability of the UE.

-	The UE discovers the data channel capability of the network.

When the UE supporting IMS data channel registers on the IMS network and the UE is allowed to use IMS data channel as specified in clause AC.4, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its IMS data channel capability.

If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request as specified in clause 9.2 of TS 24.186 [X], which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE may receive a Feature-Caps header field indicating its data channel capability in the 200 OK response to a subsequent REGISTER request when the network starts supporting IMS data channel after successful initial registration of the UE.

When the UE supporting IMS data channel initiates an IMS session, and if the UE is allowed to use IMS data channel as specified in clause AC.4, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial INVITE or any re-INVITE request message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag as specified in TS 26.114 [76] in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the data channel capability indication in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use IMS data channel, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

END OF CHANGES