3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401014

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Architectural Assumptions and Requirements for DualSteer

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: Architectural Assumptions and Requirements for DualSteer.

1. Introduction/Discussion

This contribution proposes Architectural Assumptions and Requirements for DualSteer aspects of FS_MASSS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change * * * *

4.1.1	Architectural Assumptions for DualSteer

The following architectural assumptions for DualSteer are made in this study:

-	Solutions are expected to demonstrate not to impact PLMNs that do not support this functionality.

-	A DualSteer Device either supports non-simultaneous transmission in case of DualSteer Device with a single UE, or supports simultaneous transmission in case of DualSteer Device with two separate UEs.

-	A DualSteer Device supports two subscriptions, i.e. two SUPIs, from the same operator that share one subscription profile.

-	For any particular service, at any given time, the Dual Steer Device shall transmit all traffic of that service using only a single 3GPP access network.

-	There is no coordination in RAN between the two 3GPP access networks to which the Dual Steer Device is accessing simultaneously.

-	For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

-	The solution for scenario where the DualSteer device is registered on two 5GC is the baseline for the scenario where the DualSteer Device is registered on 5GC and on EPC.

-	The DualSteer Device will not use the same SUPI to connect to two 3GPP access networks simultaneously.

-	No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

-	No impacts on RAN and UICC apps.

# * * * * Second change * * * *

4.2.1	Architectural Requirements for DualSteer

The following architectural requirements for DualSteer are made in this study:

-	Impact on the existing PDU Session management mechanism is minimized.

-	Impact on the existing registration procedure for each subscription is minimized.

# * * * * End of changes * * * *