3GPP TSG-SA WG2 Meeting #160 Ad hoc-e	S2-2401023

22–29 Jan. 2024, Emeeting

Source:	Xiaomi

Title:	WT#1 DualSteer architecture assumptions

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: The contribution proposes architecture assumptions and principles for WT#1 DualSteer to FS_MASSS TR 23.700-54.

1. Introduction

This contribution proposes architecture assumptions and principles for WT#1 DualSteer to FS_MASSS TR 23.700-54.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-54-000.

* * * Start of change * * * *

4.1.1	Architectural Assumptions for DualSteer

The following architecture assumptions are applied to the DualSteer study:

-	Solutions are expected to demonstrate not to impact VPLMNs and/or HPLMNs that do not support this functionality.

-	No coordination is assumed in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

-	For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

-	Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

-	For any particular service, at any given time, the DualSteer Device shall transmit all traffic of that service using only a single 3GPP access network.

-	No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

-	The traffic policies are in full control of the HPLMN.

-	For traffic switching, assuming data anchoring in the HPLMN. In case of PNI-NPN plus PLMN scenario, assuming data anchoring in the PNI-NPN.

-	If the DualSteer Device is a single UE, non-simultaneous data transmission is assumed over two 3GPP access networks. If the DualSteer Device is two separate UEs, simultaneous data transmission is assumed over the two networks.

* * * End of change * * * *