3GPP TSG-SA2 Meeting #160AH-e	S2-2401121

# 22 – 29 January, 2024, Electronic meeting				 (revision of S2-240xxxx)

Title:	[draft] LS reply on UE Location Information for NB-IoT NTN

Response to:	S2-2400020 (R2-2311326) LS on UE Location Information for NB-IoT NTN

Release:	Rel-17

Work Item:	IoT_NTN_enh

Source:	SA2

To:	RAN2

Cc:	CT1, RAN3

Contact Person:

Name:	Yunjing Hou

E-mail Address:	houyunjing@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for their LS on UE Location Information for NB-IoT NTN.

SA2 discussed the questions and provided the following answer:

UE location reporting can be supported by LCS Deferred EPC-MT-LR for periodic and triggered location procedure as described in clause 9.1.19 in TS 23.271. In this procedure, when the UE detects a trigger or periodic event happens, it reports the event and the available location to MME via LCS MO-LR message. This function can be applied even if LPP is not supported by the UE. Furthermore, the security can be ensured by NAS layer and the privacy will be checked by EPC when invoking the deferred EPC-MT-LR procedure.

If the procedure mentioned in the answer above cannot fulfil the location reporting requirement, SA2 thinks it is a new requirement needs to be discussed in SA1 firstly. SA2 can discuss the requirement based on SA1 conclusion in Rel-19 in FS_5GSAT_ARCH_Ph3 SID or a new TEI19.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to take the above information into account.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #161		26 February - 01 March 2024			Athens, GR

TSG-SA2 Meeting #162		15 - 19 April 2024			             (TBC), China