3GPP TSG-SA2 Meeting #160AH-e	S2-2401123

22 - 29 January 2024, Electronic Meeting	(revision of S2-24xxxxx)

Source:	CATT

Title:	23.700-59: New KI for FS_UAS_Ph3 WT#2

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3

Abstract of the contribution: This paper is to propose new key issue for FS_UAS_Ph3.

1. Proposal

It is proposed to agree the new key issue into TR 23.700-59 based on the following WT#2 of FS_UAS_Ph3.

-	WT#2: Based on SA1 requirements, study whether and how to enable network-assisted/ground-based mechanism for DAA (Detect And Avoid) that leverages information collected and generated in the 5GS, including whether and what new information is needed.

NOTE 1:	The solution shall co-exist with and leverage, to the extent possible, Direct DAA solutions considered in Release 18.

NOTE 2:	Sensing related information is out of scope of this study.

* First change *

# 5	Key Issues

## 5.x	Key Issue #x: DAA Enhancement based on information from 5GS

### 5.X.1	Description

DAA mechanisms described in clauses 5.6 and 5.7 in TS 23.256 are based on PC5 reference point without leveraging information collected and generated in the 5GS. This key issue aims to study DAA enhancements by leveraging the information from 5GS.

The following aspects need to be addressed for this key issue:

-	Whether and what new information generated in 5GS can be leveraged by DAA.

-	Whether and how to enhance the existing DAA mechanism by leveraging the information generated in the 5GS

-	Which NF and how to provide the information.

-	How does UAV and AAM use the information.

* End of change *