3GPP TSG-SA WG2 Meeting #160-Ad Hoc-e	S2-2401491

E-meeting, January 22 – 29, 2024

Title:	Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

Response to:	LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2400009/C1-237897)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Qiang Deng

Tel. Number:

E-mail Address:	dengqiang1 AT catt DOT cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR0404

1. Overall Description:

SA2 would like to thank CT1 on the LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2400009/C1-237897).

SA2 has discussed the questions and would like to provide the answers as followings.

- As specified in TS 33.503 clause 6.1.3.3.3.2, for 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE.

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

SA2 answer to Question: SA2 has discussed this matter and agreed the attached CR to resolve this issue.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take the above answers into account and develop the corresponding stage 3 specifications.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #161	                26 February - 01 March 2024 	       Athens

TSG-SA2 Meeting #162	                15 April - 19 April 2024 	       	China (TBC)