SA WG2 Meeting #160-Ad Hoc-e 		S2-2401584

E-meeting, January 22 – 29, 2024

Title:	Reply LS on Issues related to user consent for retrieving data stored in the ADRF/NWDAF

Response to:	LS C3-235567/ S2-2400044 on Issues related to user consent for retrieving data stored in the ADRF/NWDAF from CT3

Release:	Rel-17

Work Item:	eNA_Ph2

Source:	SA2

To:	CT3, SA3

Cc:	-

Contact person:	Yannick Lair

Yannick.lair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.288 CR 1023r2, TS 23.288 CR 1024r1

# 1	Overall description

SA2 thanks CT3 for their LS in C3-235567 on Issues related to user consent for retrieving data stored in the ADRF/NWDAF.

SA2 would like to provide the following answers to CT3 questions:

EXAMPLE:	NWDAF1 checks that user consent is granted for the purpose of "Analytics", so it collects and stores the data into the ADRF. NWDAF2 later retrieves this data from the ADRF and (since there is no user consent check for the interaction with the ADRF) uses it also for any other purposes, e.g. for ML Model training, although the user consent did not exist for that purpose when the data was collected and stored in the ADRF by NWDAF1, which means the NWDAF2 with unchecked purpose can bypass the restriction of the user consent.

Question 1:	Whether all the NF service consumers need to check user consent to be granted for all its usage purposes before retrieving data from the ADRF/NWDAF?

When NWDAF or DCCF intends to retrieve data from ADRF (via Nadrf_DataManagement) or from another instance of NWDAF (via Nnwdaf_DataManagement), the NWDAF or DCCF needs first to check user consent from UDM to check if data retrieval from ADRF or NWDAF is allowed as per user consent, based on the purpose for NWDAF or DCCF to retrieve data, e.g. for analytics or model training. Note that checking of user consent for data collected by an NWDAF Data Consumer via DCCF is described in TS 23.288 clause 6.2.6.3.3.

It is SA2 view that the user consent at the time of initial data collection which led data to be stored in ADRF is not relevant, what matters is the user consent at the time of data retrieval from ADRF.

Question 2:	If the answer to question 1 is yes, how to handle the user consent checking when multiple consumers retrieve the same data from the ADRF/NWDAF for the same purpose or for different purposes?

See answer to question 1. In the EXAMPLE provided by CT3, if NWDAF1 still has data that was collected while user consent was provided, and then user consent is revoked (which means NWDAF2 cannot retrieve related data from ADRF), it is SA2 understanding that NWDAF1 will handle the data as per TS 33.501 requirements in Annex V clause V.4:

“Upon notification of user consent revocation, NFs (processing the data pertaining to the revoked consent) shall halt processing and collection of the data.

Upon notification of user consent revocation, NFs may delete, quarantine, or temporarily retain the data pertaining to the revoked user consent based on local policies and legal constraints.”.

# 2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above information and attached SA2 agreed CR into account.

# 3	Dates of next SA2 meetings

SA2#161	26th Feb– 1st Mar 2024 		Athens, GR

SA2#162	15th – 19th April 2024 		China