3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401587

Electronic Meeting, 22-29 January 2024

# Title:	Reply LS on uniqueness of ProSe U2N RSC

# Response to:	LS on uniqueness of ProSe U2N RSC (S3-235076/S2-2400026)

# Release:	Release 17

# Work Item:	5G_ProSe

Source:	SA2

To:	SA3

Cc:	-

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.304 CR 0370

1. Overall Description:

SA2 would like to thank SA3 for the LS on uniqueness of ProSe U2N RSC (S3-235076/S2-2400026).

Regarding the questions from SA3:

SA3 kindly asks SA2 whether the RSC could be globally unique across different PLMNs.

SA2 would like to provide the answers as follows:

- TS 23.304v17.8.0 clause 5.8.3.1 defined the RSC as follows:

  - Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to, and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

- Additionally, the use of the RSC in the UE-to-Network Relay discovery was clarified in clause 6.3.2.3.1:

  - The Relay Service Code (RSC) is used in the 5G ProSe UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe UE-to-Network Relay provides to the 5G ProSe Remote UE. The RSCs are configured on the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE as defined in clause 5.1.4. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service based the policy as specified in clause 5.1.4. A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

- In the 5G ProSe UE-to-Network Relay architecture, as defined in clause 4.2.7, for both Layer-3 and Layer-2 operations, the Remote UE and the UE-to-Network Relay may have different HPLMNs and may register to different serving PLMNs.

Therefore, in order for these Remote UE and Relay UE to operate properly, the RSC value used need to be unique across these involved PLMNs.

In addition, SA2 has agreed a Rel-18 CR (see attached) to add clarifications to TS 23.304 regarding the use of the same Relay Service Code by UEs from different PLMNs.

2. Actions:

To SA3:

ACTION: 	SA2 would like to ask SA3 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26 February-1 March 2024	Athens, Greece

TSG-SA2 Meeting #162		15-19 April 2024	China (TBD)