3GPP TSG-SA WG2#160-Ad Hoc-e	S2-2401625

Electronic Meeting, 22-29 January 2024	(was S2-2401345r01)

Source:	Qualcomm Incorporated, Sony

Title:	Updated SID on Vehicle Mounted Relays (VMR) Phase 2

Document for:	Discussion/Approval

Agenda Item:	30.1

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: Updates to the SID for Vehicle Mounted Relays (VMR) Phase 2.

TSG SA Meeting #SP-102	SP-231799

11 - 15 December 2023, Edinburgh, Scotland	(revision of SP-231770/1676/1295/S2-2311898)

# Source:	vivo (Moderator)

# Title:	New SID on Architecture Enhancements for Vehicle Mounted Relays (VMR) Phase 2

# Document for:	Approval

# Agenda Item:	7.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Vehicle Mounted Relays Phase 2

Acronym:	FS_VMR_Ph2

Unique identifier:	1020067

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 930021 | Stage 1 of Vehicle-Mounted Relays | Stage 1 for VMR in Rel-18 |
| 980019 | Architecture Enhancements for Vehicle Mounted Relays | Stage 2 for VMR in Rel-18 |
| 850009 | Architecture enhancements for the support of Integrated access and backhaul (IAB) | Baseline IAB architecture support. |
| 941009 | Mobile IAB (Integrated Access and Backhaul) for NR | RAN aspects of the VMR feature in Rel-18. |
| TBD | Study on additional topological enhancements for NR | RAN aspects of the VMR feature in Rel-19 |

# 3	Justification

In Rel-18, SA2 conducted a study followed by a work item on VMR (Vehicle-Mounted Relays). During the study, it was agreed to limit the scope of the work to the IAB (Integrated Access and Backhaul) architecture, whereby the IAB-node consist of an IAB-MT and an IAB-DU, with the IAB-DU establishing an F1 interface with a donor CU over a wireless link.

There are however other architecture options to achieve the functionality of vehicle-mounted relay as defined in TS 22.261, for instance the so-called “Velcro” solution whereby the relay node consists of a UE co-located with a full gNB, with the gNB in the relay establishing N2 and N3 interface to an AMF and UPF residing in the 5GC over a PDU session or PDU sessions.

This architecture option which has not yet been studied by SA2 may be better suited to certain deployment scenarios for vehicle-mounted relays, for instance using the VMR where IAB is not widely supported, or the scenario in which the relay process local traffic in the vehicle to provide onboard services with low latency.

This study has high dependency with RAN aspects and coordination with RAN is needed.

Additionally, the following other functionalities were not included in the Rel-18 work:

- NTN backhauling, to provide coverage e.g. on vessels, aircrafts and in other areas without TN coverage

- a scenario where the backhaul (midhaul) MNO is different from the VMR provider.

It is therefore proposed to further study the above-mentioned items which were not specified in Rel-18.

# 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements to further evolve the functionalities of base station relays mounted on vehicles. Specifically, the objectives include:

- WT-1: Identify gaps for supporting the architecture with a gNB onboard of a relay and the use of a PDU session for the wireless backhauling of the N2/N3 interfaces e.g. including UE access control, including mobility aspects.

- WT-2: Identify whether and how to enhance the architecture to enable authorization and configuration of relay

- WT-3: Identify how to enhance the architecture to enable support for UE location services (with focus on the identification of the location of the Cell of the VMR) and emergency services

Note 1:  Coordination with RAN is needed due to the dependency with RAN.

Note 2: The wireless backhauling of N2/N3 may use the existing TN and NTN.

Note 3: Support of the multi-hop routing within the IAB topology will be handled as alignment with RAN WG inputs.

Note 4: it is assumed that Cell ID/TAC management will be studied in RAN WGs.

Note 5: In Rel-19, no enhancement is aimed to the existing QoS mechanisms for the wireless backhaul.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 1.5 | 1.5 | Yes | - |
| WT#2 | 1 | 0.5 | Maybe | Depends on WT#1 |
| WT#3 | 0.5 | 0.5 | Maybe | Depends on WT#1 |

Total TU estimates for the study phase: 3

Total TU estimates for the normative phase: 2.5

Total TU estimates: 3 + 2.5= 5.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-06 | Study on architecture enhancements for vehicle-mounted relays Phase 2 | TSG#104 Jun. 2024 | TSG#104 Jun. 2024 | Secondary Rapporteur: Lars Nord (Sony) |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Hong Cheng (Qualcomm), hongc@qti.qualcomm.com

Secondary Rapporteur: Lars Nord (Sony), Lars.Nord@sony.com (in charge of the TR for the study phase.)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-     SA3 for Security aspects

-     SA5 for charging and management aspects

-     RAN aspects

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

Dish Network

IIT Bombay

FirstNet

NEC

Nokia

Nokia Shanghai Bell

AT&T

Verizon

Xiaomi

LG Electronics

CATT

Sony

Ericsson