SA WG2 Meeting #160-Ad Hoc-e	S2-2400132

# January 22 – 29, 2024, Elbonia

Source: 	NTT DOCOMO

Title: 	KI#3: NF Abnormality analytics

Document for: 	Approval

Agenda Item: 	19.15 (FS_AIML_CN)

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes a KI for providing NF abnormality analytics in the context of WT3 of FS_AIML_CN SID.

# 1	Discussion

According to the WT3.2 of the FS_AIML_CN SID (SP-231800), the following aspects need to be studied.

WT3.2 - Study prediction, detection, prevention, and mitigation of network abnormal behaviours i.e. signalling storm with the assistance of NWDAF.

Accordingly, it is proposed to enhance the functionalities of NWDAF to provide analytics that assist prediction, detection, prevention, and mitigation of network abnormal of network functions.

# 2 Proposal

It is proposed to update the TR according to the following text.

# 5	Key Issues

********** First Change (all new text) **********

## 5.X	Key Issue #X: NF Abnormality analytics

This KI is related to the WT#3.2.

Analytics by NWDAF can assist to detect (and then mitigate) and/or predict (and consequently prevent) abnormalities in NF behaviours. Currently, NWDAF provides “NF load analytics” and analytics pointing to “suspicion of DDoS attack” but further study is needed to identify other potential additional input data and output analytics that are beneficial for detection and prediction of abnormal behaviour of network functions. This KI will study the following aspects:

-	Whether and how to enhance existing analytics or introduce a new one to generate NF abnormal behaviour analytics including statistics and predictions?

-	Which consumer NFs can subscribe to the NF abnormal behaviour analytics and how to use to mitigate or prevent the abnormalities?

-	What are the signaling storm input information needed in addition or in conjunction with “Suspicious of DDoS attack” analytics for the NF abnormal behaviour analytics and how to gather it?

-	What output information and under which conditions may be provided as the NF abnormal behaviour analytics to assist prediction, detection, prevention, and mitigation of signalling storm?

********** End of Changes **********