3GPP TSG-WG SA2 Meeting #160-AH	S2-2400087r05

# Online, January 22 - 29, 2024

Source: 	China Mobile, Rakuten Mobile, CATT

Title: 	New Solution: Policy control for network energy saving

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: Propose a solution on KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria.

1	Discussion

This paper is to propose a new solution on KI#2.

This solution provides a framework for energy saving related policy control. EECF determines energy saving policy based on EC information, UE subscription or AF request, and indicates 5GC NFs (e.g., AMF, SMF, PCF) to enforce the energy saving policy.

This paper try to merge S2-2400930 as following:

- S2-2400930 defines a new NF, EMF(Energy Management Function). The functionality of EMF is similar to EECF(Energy Efficiency Control Function), and it is proposed to use EECF as the name of the new NF.

- The maximum energy proposed in S2-2400930 is the limit of energy consumption, and energy saving policy will be enforced if exceeded. This has the same meaning of the energy consumption threshold of this paper.

- According to S2-2400930, per UE or per PDU session granularity may be considered. This is merged into step 1.

- Step 0a and steps 2-3 of S2-2400930, energy consumption is provided by UDM. This is merged into step 3 and 5.

- Steps 4-10 of S2-2400930 on information collection will reference to KI#1 solution thus not detailed described here. Granularity of energy related information collection and per S-NSSAI from OAM, proposed by S2-2400930, are merged into step 4.

- Step 11 of S2-2400930, the EECF(EMF) calculates energy consumption, which is merged to step 5.

- According to step 1 and steps 12-14 of S2-2400930, the EECF(EMF) sends energy consumption information to PCF, and PCF determines energy consumption threshold is exceeded and decides new rule. This enhances PCF functionality to enable PCF handling "raw" energy consumption related information, which causes: 1) big impact on PCF, 2) energy consumption related information is managed by two NFs. Steps 5-8 of S2-2400087 solution have no impact on PCF functionality or AM/SM policy association modification procedures, thus is preferred.

2	Proposal

It is proposed to include the below changes into TR 23.700-66 v0.2.0.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Policy control for network energy saving |  | X |  |  |
| … |  |  |  |  |

NEXT CHANGE (all the text is new)

6.x	Solution #X: Policy control for network energy saving

6.x.1	Key Issue mapping

This solution is related to the KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. This solution addresses the following issues of KI#2:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Answer: Energy saving authorization information is added in UE subscription information, authorizing enforcement of energy related policies. Energy consumption threshold may also be included in subscription information to limit the maximum energy consumption.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	Answer: This solution re-uses existing AM policy and SM policy and reuses the create/update procedures of the policies,  and PDU session release information as new SM policy parameter.

-	At what granularity (e.g., network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

-	Answer: Energy policy control is performed at UE granularity or PDU session granularity in this solution.

-	What network energy related information is required for subscription and policy control and how it is obtained.

-	Answer: Energy consumption threshold and energy consumption related information is required. The energy consumption threshold is provided by AF or UDM, and the energy consumption related information is collected from 5GC NFs and/or OAM.

6.x.2	Functional Description

This solution The architecture of this solution is described as Figure 6.x.2-1.

AMF

SMF

EECF

AF

PCF

DN

UPF

NEF

(R)AN

UE

Naf

Nnef

Npcf

Neecf

Nsmf

Namf

Figure 6.x.2-1: Architecture assumption

A new NF, Energy Efficiency Control Function (EECF) is introduced. The EECF includes at least the following functionality:

-	Collection of the information that have impact on energy consumption/efficiency per UE or per PDU session from other 5GS NFs.

-	Obtaining energy saving authorization information and energy consumption threshold via UE subscription.

-	Generating and sends energy control information to the PCF as assistance information for policy adjustment for energy saving.

Editor's note: The name of the NF, e.g. whether it is called as EECF or EMF, will be harmanized and determined in conclusion phase.

This solution performs energy saving in UE or PDU session granularity, by adjusting the AM policy and SM policy. The following existing or new parameters of AM policy or SM policy are to be sent or updated for energy saving consideration:

For AM policy:

-	UE-AMBR: Setting lower UE-AMBR limits the bit rate per UE granularity, therefore reduces the energy consumption.

-	UE-Slice-MBR: Setting lower UE-Slice-AMBR limits the bit rate per UE, therefore reduces the energy consumption.

For SM policy:

-	QoS information: Energy consumption can be reduced by enforcing QoS with lower Session-AMBR, higher PDB, higher PER, etc..

-	PDU session release: Releasing one or more PDU session(s) saves the energy consumption of the PDU session(s).

NOTE:	It is assumed that the controlled resources (e.g., Non-GBR QoS Flows, slices, PDU Sessions) are used only for the best-effort traffic and the guaranteed traffic is not affected.

In this solution, the EECF generates the energy control information as assistance information for policy adjustment for energy saving, indicates it to the PCF to adjust the parameters in AM and SM policies as described above, based on energy consumption threshold per application from the AF or UDM, energy related subscription information of the UE and energy consumption related information (e.g. data volume, bit rate, etc.) from 5GC NFs and/or OAM. The PCF reuses existing AM policy association modification and SM policy association modification procedures to update the energy saving related policies. It is also assumed that EECF can also obtain the data volume of one single PDU session through UPF event exposure service, then calculate PDU Session energy consumption.

6.x.3	Procedures

4. Energy information collection

3. Nudm_SDM_Get

2. Neecf_EnergySaving

NEF

UDM

AF

EECF

PCF

UPF

SMF

AMF

9. N4 Session Modification

8. Steps 3-5 of Figure 4.16.5.2-1 of TS 23.502

10. SMF triggers PDU Session release

7. Steps 2-4 of Figure 4.16.2.1.1-1 of TS 23.502

5.Npcf_AMPolicyAuthorization Create/Update

6.Npcf_PolicyAuthorization Create/Update

1. Nnef_EnergySaving

Figure 6.x.3-1: Procedure for policy control for network energy saving

1.	AF creates a request for energy saving, including target UE identifier(s) or S-NSSAI/DNN or Application ID, and energy consumption threshold, and sends the request to NEF.

2.	The NEF invokes Neecf_EnergySaving to send the information provided by AF in step 1 to EECF or the UDM.

3.	The EECF obtains energy saving authorization information of the target UE(s) via subscribed information from UDM. The EECF may also obtain energy consumption threshold from UDM.

4.	For the target UE(s), or PDU Session(s) of the S-NSSAI/DNN, or the application, the EECF collects energy consumption information from OAM, or collects energy consumption related information (e.g. data volume, bit rate, etc) from 5GC NFs . The details can be referred to solution of KI#1. For the data volume, the EECF can obtain the data amount of one single PDU session through UPF event exposure service.

5.	Based on the energy consumption (or related) information, and the energy consumption threshold provided by AF and/or UDM, the EECF sends energy control information as assistance information for policy adjustment by the PCF to adjust AM policy including the following parameters per target UE: UE-AMBR, UE-Slice-MBR. The EECF sends the information to PCF via Npcf_AMPolicyAuthorization Create/Update service.

6.	Based on the energy consumption (or related) information and the energy consumption threshold, the EECF sends energy control information as assistance information for policy adjustment  by the PCF to perform SM Policy Association Modification. The EECF sends the information to PCF via Npcf_PolicyAuthorization Create/Update service.

Editor's note: How the energy control information as assistance information for policy adjustment satisfies the AF request is FFS.

7.	The PCF reuses steps 2-4 of the AM Policy Association Modification procedure as specified in clause 4.16.2.1.1 of TS 23.502 [3] to update AM policy, including: e.g., UE-AMBR, UE-Slice-MBR received from EECF in step 5.

8.	The PCF reuses steps 3-5 of the SM Policy Association Modification procedure as specified in clause 4.16.5.2.1 of TS 23.502 [3] to update SM policy, including QoS parameters and indication of PDU Session release received from EECF in step 6.

NOTE:	It is assumed that the controlled resources (e.g., Non-GBR QoS Flows, slices, PDU Sessions) are used only for the best-effort traffic and the guaranteed traffic is not affected.

9.	The SMF triggers N4 Session Modification procedure specified in clause 4.4.1.3 of TS 23.502 [3] to enforce the QoS parameters in the updated SM policy in step 8.

10.If the updated SM policy in step 8 includes indication of PDU session release, the SMF triggers PDU Session release procedure specified in clause 4.3.4.2 of TS 23.502 [3].

Editor's note: whether a new NF or a NWDAF is used is FFS.

Editor's note:	Whether the EECF provides energy control information or policy adjustment information (in this case the EECF can be collocated with the PCF) is FFS.

Editor's Note: how per UE energy consumption can be derived from data on data-rate or volume is FFS (same rate or volume can consume different amount power/energy depending on radio conditions). More details on the PDU /session granularity energy related information is FFS.

Editor's Note: whether the operator can trust a third party to provide energy related thresholds for subscribers is FFS

6.x.4	Impacts on existing services, entities and interfaces

EECF: New network functionalities are as description of clause 6.x.2.

NEF: New service operation Nnef_EnergySaving.

PCF: Updating AM policy and SM policy based on the policy adjustment information received from EECF.

AMF: Updating periodic registration update timer based on AM policy.

UDM: Storing and providing energy saving authorization information and energy consumption threshold in subscription information.

UPF: Reporting the data volume of specific PDU session based on EECF request.

End of CHANGE