SA WG2 Meeting #S2-160AHE	S2-2400115R02

22 - 29 January, 2024, Electronic

Title:	[Draft] Reply LS on Coordination of work for UAS_Ph3

Response to:	LS on Coordination of work for UAS_Ph3 (S2-2400074/S6-234027)

Release:	Rel-19

Work Item:	UASAPP_Ph3, FS_UAS_Ph3

Source:	SA2

To:	SA6

Cc:	SA

Contact person:	LaeYoung Kim

laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA2 would like to thank SA6 regarding the LS on Coordination of work for UAS_Ph3 and provide the feedback as below.

The SID for Study on Phase 3 for UAS, UAV and UAM (FS_UAS_Ph3), SP-231801 approved at SA#102 includes the following work task related to UAV flight path monitoring. So, SA2's understanding is that the SA6 objective related to real time UAV flight path monitoring may have some overlap with the SA2 objective related to WT#1.

- WT#1: Based on SA1 requirements and input from aviation fora, study whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for i.e. pre-mission flight planning, in-mission flight monitoring, C2 communication reliability, interfacing with UTM (e.g. supporting the scenario of multiple USS serving the geographical areas corresponding to UAV flight path).

The CR attached in the SA6 LS (S6-233453) reads the interaction between the UAS application specific server and the UAE server, and between the UAE server and the UAE client to support real time UAV flight path monitoring where there is no direct involvement of RAN or any 5GC NFs such as NEF. Therefore, SA2's understanding is that no overlap with SA2 work is expected from a mechanism perspective.

SA2 believes that it is acceptable to develop and specify both application layer based mechanisms in SA6 and 5GS based mechanisms in SA2 as long as the two mechanisms are not conflicting . SA2 has just started FS_UAS_Ph3 at this meeting and its completion target date is TSG#104 (June 2024).Therefore, SA2 cannot assess whether there is any dependency between SA2 and SA6 mechanisms. At this stage, SA2 has no issue with progressing the SA6 CR if the contents can meet the privacy requirements if any. Later, if SA2 finds any aspect or issue that needs revisiting the contents in the SA6 CR or communicating with SA6, SA2 will inform SA6 of it.

2	Actions

To SA6:

SA2 respectfully asks SA6 to take the above feedback into account in SA6 work.

3	Dates of next TSG-SA WG2 meetings

TSG SA WG2 Meeting #161			February 26 – March 1, 2024		Athens, Greece

TSG SA WG2 Meeting #162			April 15 – 19, 2024			China (TBD)