3GPP SA WG2 Meeting #160-Ad-Hoc-e	S2-2400317r04

e-meeting January 22-29th, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #2: New Sol: Control plane architecture and procedure for S&F

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a solution to KI#2: Support of Store and Forward Satellite operation.

# Introduction

This contribution proposes EPC architecture enhancement to support S&F operation.

# Discussion

The proposed architecture is defined by considering the minimum necessary set of network element(s) onboard satellite to support S&F operation. The S&F operation is suitable for the delivery of delay-tolerant services.

The Store and Forward Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. The end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time.

The proposed solution represents UE procedure for attach and CIoT EPS optimization in S&F scenarios.

# Proposal

A solution is proposed for KI#2 for incorporation in the FS_5GSAT_ARCH_Ph3 TR23.700-29.

*** Start of changes (all new text) ***

## 6.X	Solution #X: MME Split Architecture to support Control plane procedure for S&F

### 6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #2, " Support of Store and Forward Satellite operation ".

### 6.X.2	Description

In this proposal, the MME, which is the mobility anchor for the UE, is split into 2 network elements, MME-NT (MME on board satellite) and MME-T (MME terrestrial). The MME-NT shall act as a mobility anchor point to UE and does only subset of MME functionality such as assist in Store and Forward operations i.e. combination of two steps not concurrent in time. The MME-T on the ground is responsible for maintaining UE context, including security context. MME-T is also responsible for NAS ciphering/deciphering and integrity protection. MME-NT is responsible for maintain S1 connection towards RAN node and maintain associated connection ID per UE towards MME-T. MME-NT is responsible for encoding NAS message received from MME-T into S1AP payload towards RAN and decode NAS payload from messages received from RAN and send it to MME-T.

Figure 6.X.2-1: Architecture enhancement for control plane in Store and Forward operation

Figure 6.X.2-2: Example of split MME architecture based on UE coverage or ground station connectivity at different time.

### The Store and Forward Satellite (S&F) operation in a EPC system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

### Figure 6.Y.2-1: Example of MME split architecture for Store and Forward operation

### In regenerative architecture, the RAN will be placed in a satellite. In the Store & Forward (S&F) scheme, the MME is at satellite will be responsible for providing Store and forward feature. MME in satellites are represented as MME-NT (non-terrestrial) in the above diagram. Let’s assume the time T1 is between 10:00 and 10:20 and T2 is between 10:40 and 11:00. The satellites will have ground station coverage when hovering over Rennes and Orleans but not when crossing Le Mans.

### At time T1, the black satellite covers Le Mans, and the white satellite provides coverage to Rennes. At T2, the black satellite will be at Orleans and the white satellite will be covering Le Mans.

At time T3, the white satellite will sync up with the MME-T at Orleans.

### 6.X.3	Procedures

#### 6.X.3.1	Architecture enhancement to support S&F

In regenerative architecture, the RAN will be placed in Satellite. In the Store & Forward (S&F) scheme, the MME is at Satellite will be responsible for providing Store and Forward feature. MME in the satellite is represented as MME-NT-1 (non-terrestrial) in the above diagram. E.g., in above figure 6.X.2-2 for the time T1 is between 10:00-10:20 and T2 is 10:30-10:50 and the satellite at T1 is providing coverage towards UE but not connected to the ground station, it shall store UL messages and forward DL messages towards UE; When the satellite at T2 is not providing coverage for UE but connected to ground station shall store any DL messages for the UE and forward any UL messages it had received at T1. At T3, the MME-T can select the appropriate MME-NT-2 in satellite for storing DL messages meant for the UE.

Figure 6.X.2-1: Architecture enhancement for control plane in Store and Forward operation

- MME-T manages all the existing functionality of existing MME. Instead of maintaining S1AP ID per UE context, it will maintain a context identifier for the S10` messages towards MME-NT

- MME-T is responsible for NAS ciphering/integrity protection function.

- MME-T needs to forward the RAN S1AP ID towards MME-NT to help MME-NT to maintain UE context per S1AP instance.

#### Editor’s note: It is FFS on what information the MME-NT is required to store for control plane procedure.

#### 6.X.3.2	Attach Procedure in EPC in S&F scenario

Figure 6.Y.3-2: Attach procedure in S&F

1.	A UE, camping on an E-UTRAN cell reads the related System Information Broadcast. If the UE can proceed to attach, it initiates the Attach procedure by the transmission, to the eNodeB, of an Attach Request (IMSI or old GUTI, Old GUTI type, last visited TAI (if available).

2.	The eNodeB(RAN-1) forwards the Attach Request message in a S1-MME control message (Initial UE message) towards MME-NT-1. In the case of satellite access for Cellular IoT, the MME-NT-1 may verify the UE location and determine whether the PLMN is allowed to operate at the UE location.

3.	If the MME-NT-1 is not in contact with the ground station when receiving a message in step2, MME-NT-1 shall store the attach request message and, generate a interim GUTI and send an new NAS clear text message towards UE asking to save the interim GUTI for future NAS transactions. MME shall also provide the validity time for this interim GUTI. (UE may send an ack for the same, not shown in the diagram).

4.	When MME-NT-1 regains ground connectivity, it shall forward the attach request, IMSI along with the interim GUTI created for this request in step3 towards MME-T. A UE having valid interim GUTI shall not reset the MM context.

5	If no UE context for the UE exists at the MME-T, and if the Attach Request (sent in step 1) was not integrity protected, or if the check of the integrity failed, then authentication and NAS security setup to activate integrity protection and NAS ciphering are carried out. The MME-T borrows the AV from HSS.

6.	MME-T, after getting the authentication key from HSS, shall try to ascertain the next available satellite which can reach the UE next. When found, it shall create the Authentication NAS payload and forward it to MME-NT-2 (the next available satellite to reach UE). MME-T shall also provide the last known location of UE.

NOTE 1: The MME-T will need the information on next suitable satellite which can serve the UE in shortest possible time. This information can be provided within the PLMN or from an external source.

6.	When MME-NT-2 reaches the UE area, it will page the UE using either the IMSI or interim GUTI or both or UE can also reach out to RAN when seeing the new cell and reattempt the attach procedure with the previously valid interim GUTI. In both cases, when the UE gets RRC connected, the MME-NT shall forward the stored (in step 6) Authentication Request message to UE.

7.	UE responds back with an authentication response towards MME-NT-2, which MME-NT-2 shall store it till it regains ground connection.

8.	When MME-NT-2 regains connectivity with the ground station, it shall forward the stored Authentication response from UE to MME-T. MME-T validates the response.

9.	If the UE is authenticated by MME-T successfully, MME-T initiates Security mode by selecting the next available satellite (as per NOTE 1) that can serve the UE next. When found, it shall relay the Security mode command to MME-NT-3 (shown as MME-NT-1 in the diagram). MME-NT-3 shall store the security mode command until it reaches the UE serving area. MME-T shall also provide the last known location of UE.

10.	When the MME-NT-3 reaches the UE serving area, it shall page the UE using the interim GUTI or IMSI or both. or UE can also reach out to RAN when seeing the new cell and reattempt the attach procedure with the previously valid interim GUTI. When the UE becomes connected, the MME-NT-3 will forward the stored security mode command message to UE.

11.	Once the UE applies the security mode, it shall acknowledge the security mode command towards MME-NT-3. MME-NT-3 shall store it till it regains the ground connection again.

12.	When the MME-NT-3 regains ground connectivity, it relays the stored security mode ack message to MME-T.

13.	After receiving the security mode ack from MME-NT-3, the MME-T selects a Serving GW and allocates an EPS Bearer Identity for the Default Bearer associated with the UE. Then it sends a Create Session Request (IMSI, MSISDN, MME-T TEID for control plane, PDN GW address, PDN Address, APN) message to the selected Serving GW.

14.	The Serving GW creates a new entry in its EPS Bearer table and sends a Create Session Request (IMSI, MSISDN, APN, Serving GW Address for the user plane, Serving GW TEID of the user plane, Serving GW TEID of the control plane) message to the PDN GW indicated by the PDN GW address received in the previous step. (not shown in this diagram and SGW+PGW are represented as SAE GW).

15.	When the create session is received from SAE-GW, the MME-T shall store till it finds the next available satellite to serve the UE next. Once found, the MME-T shall send Attach accept along with Create session response information such as user plane address and TEID to MME-NT-4. MME-T shall also provide the last known location of UE.

16.	The MME-NT-4 shall store the message until it reaches the UE serving area again. When it reaches the UE serving area, it shall page the UE using the interim GUTI, IMSI or both. UE can also connect on its own when it sees a new cell in a new satellite. In either case, when the UE comes to connected state, the MME-NT-4 shall forward the message to RAN and UE. The RAN will create its PDN resources based the user plane IP and TEID information. The UE shall receive the Attach Accept, new GUTI, along with the PDN information.

17.	UE shall acknowledge by sending the Attach complete message to MME-NT-4. MME-NT-4 shall store it till it regains the ground connectivity.

18.	When the MME-NT-4 regains the ground connectivity, it shall forward the stored Attach complete message to the MME-T.

19. After receiving Attach Complete, MME-T shall configure the PDN connection by sending a Modify bearer Request to SAE-GW by including RAN’s tunnel ID and IP information. (not shown in the diagram).

Editor's note:	How to avoid/minimize DoS attack on MME-NT while handling integrity protected messages from UE (as it lacks the NAS keys for decoding) is FFS.

Editor’s note: How to avoid/minimize fake base station attack when delivering plain text RRC message (message 3) is FFS.

NOTE 2: The UE shall support storing interim GUTI and use it to respond to Paging or while sending any initial NAS request until UE is attached to the network.

NOTE 3: MME-NT should be able to create distinct interim GUTI (one assumption is to have a distinct MME ID for each MME in satellite).

NOTE 4: MME-NTs are not keeping any security material (both AS and NAS). Integrity and ciphering are to be done at MME-T for every NAS message. This is to keep MME-NT as lightweight as possible to save precious Satellite energy resources.

NOTE 5: How to create trust between MME-T and MME-NT is up to SA3.

NOTE 6: 	Due to the dependency on paging procedures (step 6, 10 and 16) to bring the UE into connected mode for DL signalling messages, the failure in paging will prolong the attach procedure.

#### 6.X.3.3	MO Data Transport in Control Plane CIoT EPS Optimisation in S&F

High level procedure of Control plane CIoT EPS optimization in Store and Forward scenario be shown as following.

Figure 6.Z.3.1-1: High-level Procedure of S&F Operation for MO Data Transport in Control Plane CIoT EPS Optimisation

0.	The UE is ECM-IDLE.

1.	The UE establishes a RRC connection and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS Bearer ID and encrypted Uplink Data. The NAS PDU is relayed to the MME-NT-1 by the eNodeB using a S1-AP Initial UE message.

2.	The MME-NT-1 shall store the NAS payload without integrity check or deciphering until the satellite regains ground connection.

Editor’s note: In step 1, how the MME-NT creates the temporary UE context for the ciphered NAS message is FFS

3.	The MME-NT-1 sends the NAS PDU received in step 1 to MME-T when the Satellite 1 regains ground connection.

4.	The MME-T checks the integrity of the incoming NAS PDU and decrypts the data it contains.

5.	If the S11-U connection is not established, the MME sends a Modify Bearer Request message for each PDN connection to the Serving GW.

6.	If the Info IEs and/or UE Time Zone and Serving Network id are present in step 4, the Serving GW shall send the Modify Bearer Request message (RAT Type, MO Exception data counter) to the PDN GW.

7.	The PDN GW sends the Modify Bearer Response to the Serving GW.

8.	If a Modify Bearer Request message was sent at step 5 the Serving GW shall return a Modify Bearer Response (Serving GW address and TEID for uplink traffic) to the MME as a response to a Modify Bearer Request message. The Serving GW address for S11-U User Plane and Serving GW TEID are used by the MME to forward UL data to the SGW.

9.	The MME sends Uplink data to the P-GW via the S-GW.

10.	If the MME-T is not aware of pending MT traffic and S1-U bearers are not established, step 11 is skipped and step 13 applies.

11.	If Downlink data are received in step 10, the MME-T ciphers and integrity protects the Downlink data.

12.	MME-T stores the DL message until it find suitable MME-NT which can serve the UE area next.

13a-b.	Once a suitable satellite is found, the MME-T will forward the NAS DL message to MME-NT-2. If step 11 is executed then Downlink data are encapsulated in a NAS PDU and sent to the eNodeB in a S1-AP Downlink NAS Message.

14.	The MME-NT-2 shall attempt to page the UE via its own RAN on-board the same satellite. The UE responds and get RRC connected.

15.	The eNodeB sends a RRC Downlink data message including the Downlink data encapsulated in NAS PDU.

16.	The eNodeB sends a NAS Delivery indication to the MME-NT-2 if requested, MME-NT-2 shall store the “S1-AP NAS Non Delivery Indication” till it regains ground connection.

17.	When ground connectivity is available for MME-NT-2, it will deliver the stored delivery indication NAS message to MME-T.

Editor’s note: In step 12, how the MME-T finds the suitable MME-T is FFS

Editor’s note: How to handle T3417 timeout due to expected larger delay for S&F based CIoT operation is FFS.

### 6.X.4	Impacts on services, entities and interfaces

UE:

- NAS impact to handle larger delay due to S&F mode of operation.

- NAS impact due to new interim GUTI allocation

MME-T:

- Handling of new S10 messages to carry both ciphered and plain text NAS messages to/from MME-NTs

MME-NT:

- Handling of new S10 messages to carry both ciphered and plain text NAS messages to/from MME-T

- Handling of plain text NAS and NGAP messages on behalf of MME-T

*** END of changes ***