3GPP TSG-WG SA2 Meeting #160-AH	S2-2400325r09

# Online, January 22 - 29, 2024

Source: 	China Mobile, Intel, Huawei, HiSilicon, Samsung

Title: 	New key issue: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose a key issue about WT#1.1: Reducing impact on central 5GC NFs to support EAS (re)discovery and UPF (re)selection.

1	Discussion

This paper is to propose a new key issue according to the WT#1.1 of FS_eEDGE_5GC_ph3.

The FS_eEDGE_5GC_ph3 objectives for WT#1.1:

- WT#1.1. Handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

The content from S2-2400291,S2-2400718,S2-2400801,S2-2401226 are merged into this paper.

2 Proposal

It is proposed to include the below changes into TR 23.700-49 v0.0.0.

FIRST CHANGE (all the text is new)

5.X	Key Issue #X: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs

5.X.1	Description

This key issue is to investigate whether and how to reduce impact on central NFs when supporting EAS (re)discovery and UPF (re)selection.

Current Edge Computing design has impact on central 5GC Control Plane NFs. For example, the EAS Deployment Information and local UPF information (deployment information, or load) may be changed dynamically or frequently, as a consequence, they should be updated at the central SMF. Furthermore, central SMF can also be a bottleneck point for the DNS message handling since all decision is made by the central SMF. This may also cause signalling overload across operator’s data centers for a deployment scenario where Control Plane NF(s) such as SMF is deployed in the central data center while other involving NFs (e.g. L-NEF, AF, etc.) are located in the local data center.

In order to reduce the abovementioned impact on central SMF for the purpose of EAS (re)discovery and local UPF (re)selection, following aspects need to be studied:

-	Whether and which edge computing related information (e.g. UPF and EAS deployment information, DNAIs) can be locally managed to support EAS (re)discovery and UPF (re)selection.

-	How to manage the above identified edge computing related information, e.g. locally, centrally or in both places.

-	Whether existing procedures (e.g. discovery, ULCL/BP, mobility and influence/exposure related procedures) suffice. If not, how to address the gap when the above identified edge computing related information is handled locally or both locally and centrally.

End of CHANGE