3GPP SA WG2 Meeting #160-Ad Hoc-e	S2-2400368 r01

Electronic meeting,  22-29 January, 2024

Title:	[Draft] Reply LS on packet delay measurement

Response to:	LS on packet delay measurement (S2-2400013 / C4-234616)

Release:	Rel-18

Work Item:	XRM

Source:	SA2

To:	CT4

Cc:	CT3

Contact person:	Paul Schliwa-Bertling

paul.schliwa-bertling@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501, CR#5082

- Overall description

SA2 thanks CT4 for its LS on Issues on packet delay measurement in S2-2400013 (C4-234616). SA2 has discussed CT4 questions and provides answers below as follows:

1.	Whether the UPF is required (from Rel-18 onwards) to report minimal and maximum measurement result for packet delay for event-based reporting when the minimal waiting timer is over?

SA2 Reply: No, it is not. UPF is required to report only the minimal measurement monitoring result if it matches or exceeds the threshold when the minimal waiting timer is over.

2. 	If so, should the UPF send a measurement report containing minimal delay and maximal delay if the latest packet delay measured (just before the expiry of the minimal waiting timer) is smaller than the threshold?

SA2 Reply: see reply to question 1.

3.	Whether the above requirement for the UPF is applicable also for periodic reporting? If not, what are reasons?

SA2 Reply:  UPF is not required to report minimal measurement results for packet delay for event-based reporting when the minimum waiting timer is over or in periodic reports.

As result of the discussion initiated by the questions raised by CT4 in the LS, SA2 agreed to introduce stage 2 changes to TS 23.501 aligning the specification text with the answers provided in this response LS, see attached CR.

- Actions:

To CT4:

ACTION:

SA2 asks CT4 to take the information above into consideration.

- Dates of next TSG SA WG 2 meetings

SA2#161                 2024/02/26 – 2024/03/01	Athens

SA2#162                 2024/04/15 – 2024/04/19	China